Assistant’s HAT

Updated August 2019

Table of Contents

Beginning of Day and Morning Office Duties……………………………………………………………………………...	7

Changing O2 Tanks…………………………………………………………………………………………………………………...	8

Opening the Office and Morning Lab Duties……………………………………………………………………………….	8

Test Strips for Sterilizer…………………………………………………………………………………………………………….	9

Confirm In-Office Sterilizer Strip Testing…………………………………………………………………………………...	10

SURGERIES

PRF………………………………………………………………………………………………………………………………………….	11

Things to do before all surgery…………………………………………………………………………………………………..	12

First Assistant Responsibilities………………………………………………………………………………………………….	13

Perio/Crown Lengthening Surgery…………………………………………………………………………………………….	13

Tissue Grafting………………………………………………………………………………………………………………………….	14

Pinhole Surgical Treatment……………………………………………………………………………………………………....	16

Implant Surgery……………………………………………………………………………………………………………………….	17

Logging Implants……………………………………………………………………………………………………………………..  19

Solid Bite Surgery……………………………………………………………………………………………………………………..	21

NSequence………………………………………………………………………………………………………………………………. 24

Bone Expansion……………………………………………………………………………………………………………………….	25

Trichloracetic Acid…………………………………………………………………………………………………………………...	26

Returning Failed Implants………………………………………………………………………………………………………...	27

Sending Implants for Credit……………………………………………………………………………………………………...	27

Discharging patients after a surgical procedure…………………………………………………………………………	27

Clean up after a surgery…………………………………………………………………………………………………………….	28

Upper Extraction Set up…………………………………………………………………………………………………………….	29

Lower Extraction Set up…………………………………………………………………………………………………………….	30

Uncovering Set up and Procedure Steps…………………………………………………………………………………….	31

Osstell Check…………………………………………………………………………………………………………………………….	32

Quick Text for uncovering and osstell………………………………………………………………………………………...	33

Electrosurge/Radiosurge Set up………………………………………………………………………………………………..	34

Piezo………………………………………………………………………………………………………………………………………..	35

Autotome………………………………………………………………………………………………………………………………...	35

Sonic Weld……………………………………………………………………………………………………………………………….	36

Ceraroot Ceramic implant………………………………………………………………………………………………………...	37

IV SEDATION AND EMERGENCY PROTOCOL

IV Sedation by Sheldon Staff……………………………………………………………………………………………………..	38

Pre-Operative Instructions for Sedation……………………………………………………………………………………. 41

IV Sedation and Anesthesia Guidelines for Anesthesiologist……………………………………………………...	44

Set up for Anesthesiologist……………………………………………………………………………………………………....  45

Emergency Protocol………………………………………………………………………………………………………………...	46

EKG Operations……………………………………………………………………………………………………………………….	46

Running an Air Driven Handpiece if Power Goes Out…………………………………………………………………  47

Easy X-Trac system…………………………………………………………………………………………………………………..	49

Coe-Pak…………………………………………………………………………………………………………………………………..	50

Soft Reline……………………………………………………………………………………………………………………………….

PerioFlow………………………………………………………………………………………………………………………………...	50

Peri-Implantitis surgery…………………………………………………………………………………………………………...	51

Discharge of surgical patient…………………………………………………………………………………………………….	51

Contents of each cassette………………………………………………………………………………………………………….	52

Recementing Crown and /or Bridge………………………………………………………………………………………….	54

LAB WORK

Taking Impressions………………………………………………………………………………………………………………….	53

Pouring up Impressions…………………………………………………………………………………………………………...	58

Trimming Models…………………………………………………………………………………………………………………….	58

Making an essix, bleaching tray etc…………………………………………………………………………………………...	59

Hard Relines…………………………………………………………………………………………………………………………….	61

Constructing a  Face Bow………………………………………………………………………………………………………….	61

How to Make a Farley Appliance……………………………………………………………………………………………….	62

Nightguards……………………………………………………………………………………………………………………………..	62

POST OPERATIVE VISITS

POT/OHI for SB cases……………………………………………………………………………………………………………….	63

Post Op Policy and set up………………………………………………………………………………………………………….	65

POT1 for Crown Lengthening, Implant and Osseous Procedures…………………………………………………	65

POT 1 for STG and BG procedures……………………………………………………………………………………………..	66

Pinhole Post Ops……………………………………………………………………………………………………………………...	66

POT4……………………………………………………………………………………………………………………………………….	67

Re-Evaluation Appointments…………………………………………………………………………………………………….	69

OHI for EAPT…………………………………………………………………………………………………………………………….	70

Policy and Procedure for Fixed Prosthetic Cleaning and Removal……………………………………………….	70

Quick Checks……………………………………………………………………………………………………………………………  72

Desensitizing……………………………………………………………………………………………………………………………	72

Torrens Powder………………………………………………………………………………………………………………………..	73

Emergency Appointment………………………………………………………………………………………………………….	74

New Patient Procedure Start to Finish……………………………………………………………………………………….	74

Plaque Control………………………………………………………………………………………………………………………...	84

Implant Education…………………………………………………………………………………………………………………...	84

Obstructive Sleep Apnea…………………………………………………………………………………………………………..	85

EAPT process, supplies and post op………………………………………………………………………………………….	86

Oral DNA: procedure and shipping…………………………………………………………………………………………..	88

Vista………………………………………………………………………………………………………………………………………..	90

Stella Life………………………………………………………………………………………………………………………………...	91

FA checksheets………………………………………………………………………………………………………………………..	92

Updating Medical History………………………………………………………………………………………………………...	95

Guidelines for FA……………………………………………………………………………………………………………………..	95

Checking codes and Doctor Numbers………………………………………………………………………………………..	97

Sterilization (machines, cold sterile, solutions).......................................................................................................	97

Maintenance care of lines and Bottles……………………………………………………………………………………….	98

Cleaning Statim and Autoclaves………………………………………………………………………………………………..	99

Ultrasonic operation………………………………………………………………………………………………………………..	100

Purevac Maintenance……………………………………………………………………………………………………………    100

Sterilization Lab Maintenance and End of Week checksheet……………………………………………………   101

Changing Main Suction Traps………………………………………………………………………………………………..	103

Upstairs Lab………………………………………………………………………………………………………………………….	103

Sterile Gauze and C-Sponges…………………………………………………………………………………………………..	103

Receiving and Checking in Lab Cases etc………………………………………………………………………………...	103

Medication Protocol……………………………………………………………………………………………………………….  108

Adding an Rx………………………………………………………………………………………………………………………....  109

Taking x-rays with Dexis………………………………………………………………………………………………………...	110

Nomad…………………………………………………………………………………………………………………………………..	112

I-CAT: Taking a CT scan etc…………………………………………………………………………………………………….   113

Preparing Lines for CT and transferring to Invivo…………………………………………………………………...	123

Taking a Pano………………………………………………………………………………………………………………………..	116

Printing a Pano……………………………………………………………………………………………………………………...	117

Printing or Emailing Frames of CT scan…………………………………………………………………………………..	117

Copying CT to disc………………………………………………………………………………………………………………….	118

Reconstruction of CT scan (Lost CT scans).............................................................................................................	118

Calibrating CT scan………………………………………………………………………………………………………………..	119

Taking Photos with the Canon………………………………………………………………………………………………..	128

Taking Photos with Dine Corp Camera…………………………………………………………………………………....	120

Taking Photos with the Pentax………………………………………………………………………………………………..	120

Photos taken with mirrors……………………………………………………………………………………………………...	122

Downloading photos……………………………………………………………………………………………………………...	124

SNAP……………………………………………………………………………………………………………………………………..	124

Surgical Guide by an Outside Lab……………………………………………………………………………………………	125

Surgical Guide by our Office…………………………………………………………………………………………………...	125

Taking Video………………………………………………………………………………………………………………………….	126

Creating Case Photos……………………………………………………………………………………………………………..	127

Blue Note System…………………………………………………………………………………………………………………..	128

Closing Day Checksheet………………………………………………………………………………………………………….	129

Ordering Supplies…………………………………………………………………………………………………………………..	130

Opening office and Morning Lab Checksheet…………………………………………………………………………...	131

Compressor CO2 Tanks…………………………………………………………………………………………………………..	132

Switching Regulator Tanks……………………………………………………………………………………………………..  132

Laundry………………………………………………………………………………………………………………………………...	133

Handling Sharps and Biohazard materials……………………………………………………………………………….	133

Handling Drug Protocol…………………………………………………………………………………………………………..	134

Cyntox Black Waste Containers for pharmaceutical waste………………………………………………………..	146

OSHA……………………………………………………………………………………………………………………………………..	147

MSDS……………………………………………………………………………………………………………………………………..	137

Walkie Radio and Headset System…………………………………………………………………………………………..	138

Maintenance Contacts…………………………………………………………………………………………………………….	142

Office Appearance………………………………………………………………………………………………………………….	142

Banyan and Social Media………………………………………………………………………………………………………..	143

NuCalm………………………………………………………………………………………………………………………………….  145

Create New Pt. and Family File………………………………………………………………………………………………..  146

Checking Patient’s Balance……………………………………………………………………………………………………..  147

Confirming Patient………………………………………………………………………………………………………………… 148

Create New PC Sheet……………………………………………………………………………………………………………… 148

Editing Existing PC Sheet……………………………………………………………………………………………………….. 148

Printing PC Sheet…………………………………………………………………………………………………………………… 149

Document Transfer from DSN to Dentrix…………………………………………………………………………………. 149

Entering Pt. current Medication List……………………………………………………………………………………….. 150

Finding a Next Visit Appointment…………………………………………………………………………………………… 151

How to create/print Deleted Transaction Report……………………………………………………………………   152

How to Follow Up patient on Unscheduled List……………………………………………………………………….  152

Entering Details of MH with Patient………………………………………………………………………………………..  152

Perio Charting……………………………………………………………………………………………………………………….  153

Entering Hygiene Quick Text…………………………………………………………………………………………………..  154

Adding a Transaction……………………………………………………………………………………………………………..  154

Making a Next Visit………………………………………………………………………………………………………………..  154

Inputting and Updating patients MH info………………………………………………………………………………..  155

Entering Payment and Place Transaction………………………………………………………………………………..  155

Placing Document and Dentrix Document Center……………………………………………………………………. 156

Patient Education………………………………………………………………………………………………………………….. 156

Pictures of Models (for reference only).....................................................................................................................  160

# Beginning of the Day

- Set up the lab sterilization area.

  - Change the ultrasonic solutions if not done the evening before.

    - Empty the solutions in the ultrasonic unit and wipe out the ultrasonic unit.

    - Fill the ultrasonic unit with one cup of liquid ultrasonic cleaner with water added.  Add more water until full.

    - Press the “Degas” setting on ultrasonic and set for 10 minutes.

  - Cold Sterile

    - Remove sharpening stones from the cold sterile, dry off and bring to hygiene.

    - Remove the denture brushes from the first cold sterile and place in the second cold sterile storage container on the counter.

    - Remove everything, dry, wrap, and place in appropriate container.

  - Any instrument lying on towels are bagged and placed in the autoclave.

  - Run the autoclaves, statim, and Bravo.

    - Close the door and turn on.

    - Turn the autoclave on by pushing the appropriate buttons. (pouches is used for gauze and c-sponges. Packs is used for everything else)

    - All handpieces or motors are placed in the Bravo

    - Press the start button and the autoclave automatically goes through the cycle.

  - Start the laundry

  - Turn on all room lights, computers, units, Carestream laptop, and fill hot water bath and turn on.

  - Turn on oxygen in sterilization. If the regulator dial is at zero after you have turned it on, you must switch the tanks outside

  - To do this: Get the key to the oxygen tank closet from the front desk drawer. GO outside of room #3 to the oxygen tank closet. There are 3 tanks. They should be used from left to right. Meaning if the left tank’s regulator is showing empty, but the knob is turned so that you know the line is open, this tells us the tank is empty. You must turn knob to the right to close off this tank. Once this is done, move to the middle tank and open the knob by turning to the left until the dial on the regulator begins to go up. Close and lock the closet door. Confirm this is working by checking that the dial has gone up in the sterilization lab. Return key to front desk.

  - Check oxygen in each room and make sure it is working.

  - Turn on the warmer for the face towels and make sure they are stocked.  Refills are in the bottom drawers over head cabinet.

  - Turn on the blanket warmer and make sure it is stocked with blankets for patients.

Opening the Office and Morning Lab Duties

- ____Turn on O2 inside sterilization

- ____Turn on water and suction in sterilization

- ____Open all rooms 1,2,3,6, consult rooms (if they are not already open)

  - -computers/TV in rooms

  - -units in rooms

  - - water baths in rooms

  - - handpiece lines run for 2 mins

  - - water bottles full (don’t forget the bottles in rooms 2, 3, and 6 under the countertop)

4. ____Turn on CT scan and computer

5. ____Fill Ultrasonics (small U/S gets the green liquid and Large U/S gets the purple liquid)

7. ____Empty cold sterile, rinse all and put items away

8.____Make sure all autoclaves/statim have plenty of water

9. ____Empty statim jug of all water. Then refill water up to the coils

10. ____Make sure exam packs, cotton pliers, curettes, scissors are in each room (hygiene rooms if     they will be used this day)

11.____Turn on facial towelette machine and blanket warmer

12. ____ Take garbage cans out to curb on Mon and Thurs.

13.____Wrap and run any dirty instruments/cassettes

14.____On Mon: fill baby wipe warmers with 4x4 and Hibiclens

15.____ Stock sterilization fridge with ice packs

16.____ Place Wypall towels out on counters

Updated 07/15/19

# Test Strips for Sterilizers

- Pull 4 test strips envelopes out of top drawer in sterilization room

- Place the correct sticker on the envelope for that machine.

- Eleanor prints them out on her template.

- When low ask her to print some more.

- Pink card in second drawer has the model and serial numbers written down.

- Use cotton pliers to place test strip for that unit.

- Place test strip on metal rack.

- Run machine according to directions.

- When machines have finished their cycle, remove test strip with cotton pliers and place in envelope and seal.  Test strip must be completely dry before placing in the envelope or they will come back positive.

10. Mark your initials and date when test strips are done on the log sheet that is on the peg board in the sterilization room.

11. Put postage on and mail.

12. Test strips are done everyday including Thursdays.

# CONFIRM IN – OFFICE BIOLOGICAL MONITORING SYSTEM TEST VIalS

- If you get a notification that a sterilizer has failed, put a note on the sterilizer that it is not to be used!

- Pull out the incubator stored in the drawer and plug it into the wall to warm up to the right temperature. ( Leave plugged in) The unit will flash while it is warming up. It can take 15-30 minutes to warm up to the correct temperature. It should be at 60 C.

- Pull two tubes out of the box and put one tube on the rack in the machine that failed.

- Run the unit as normal.

- Let the tube cool for about 10 minutes.

- Take the tube and put in the slot to break the tube.

- Take the control tube that was not sterilized and write the letter “C “on it for control tube.

- Place that tube in the slot to break the tube.

- Place both tubes in one of the holders and let them stay there undisturbed for 3-5 hours.

- Control tube should turn from purple to YELLOW.

- Sterilized tube should stay PURPLE.

- If the test fails, call a repair guy from Benco Dental to come out to service the machine.

- If the machine passes then the machine can be used immediately.

- Write down the results in the record keeper log book.

- Throw away the tubes in the trash.

# Morning Lab Duties

See Opening Office Checksheet

PRF (Platelet Rich in Fibrin)

Set Up:

- PRF Tube holder

- Blue piece of paper

- Tourniquet and stress ball

- Two pieces of gauze

- Two 2 inch pieces of tape or coe-band

- Two alcohol prep pads

- Vacutainer butterfly and collection set

- Appropriate amount of red top tubes and white top tubes (per the doctor)

Procedure:

Only those trained should do the blood draw.

- Take vitals and record

- Confirm the procedure with the patient

- Ask the patient how they do giving blood and if there is a certain arm that is best.

- Use tourniquet and find a vein.

- Prep with the alcohol wipe

- Draw the blood using the red tops first, then white tops next if those are needed. (white tops are needed if needing to do a PRF block)

- The vials should be inserted into the centrifuge within a minute of the blood draw.

- Always draw an even amount. If you only can get 3 vials of blood, you will need to fill a 4th vial with water.

- There are 8 slots in the centrifuge, you must be sure that the centrifuge is balanced out and there are not more on one side than the other. (ie: you’ve drawn 4 reds and 2 white, the white tops will be across from each other and the red across from each other.)

- A minimum of 4 tubes are needed to to spin out. If you only get 2 tubes of blood, fill up 2 tubes of water and place them across from each other.

- If you are putting both red and white top tubes into the centrifuge, set the timer for 3 minutes. Once the 3 minutes are over, the machine will beep and you must press stop, open and remove the white top tubes , close the lid and set the machine for an additional 15 minutes.

- Once this beeps, press the stop button and remove the remaining red tops

- To prepare the PRF, visit https://www.intra-lock.com/videos.html#l-prf-videos

# Surgeries

## Things to do before all surgeries

- Have most current PA’s and/or CT scan and Invivo up on screen.

- Take digital photos of surgical site. If none have been taken.

- Ask patient if he or she would like to listen to music from Pandora(if not sedation) or if they would like a blanket.

- Patient has rinsed with Chlorhexidine.

- Face has been wiped with Hibiclens

- You have applied topical anesthetic only when Dr. comes in the room, not before.

- Use PC sheet and fill out surgery cheat sheet.  (This should be found in the top drawer next to the sinks. )

- Set up the surgery using the hat for the appropriate set up.

- Be sure doctor’s surgical gloves ( LNS – 8, MVF - 7, MES – 7 ½), masks, hats, and coats are out. Glasses for assistants are out.

- If setup for IV see page ___

- Complete Sx. Set up sheet and have out for the doctor to review

- Once patient is seated take BP and fill out “surgery cheat sheet”using the PC sheet printed

- If surgery calls for PRF, have PRF drawn at this point.

- Drape patient and put patient’s hat on, glasses and blanket if needed.

- Make sure all barriers are on light, computer, x-ray unit.

- Place topical once ready to begin surgery.

The first assistant is fully in charge of the surgery. She must:

- Make sure she has researched what we are doing and why. She has read the notes, dictations and examined PC sheet in full

- Makes sure the patient and the room is completely ready for the doctor (no matter who set it up)

- Patient is comfortable and ready (face wash, rinse, barriers, eye wear, blanket, music station selected and fully understands what we are doing today and all questions answered)

- Is able to go get doctor briefed and prep’d for surgery.

- Make sure the doctor is in the room on time . This means going out to get him or recruiting help to get him in the room

- All xrays (CT, INVIVO and DEXIS) and all designated materials are out and ready

- Is able to recruit if needed, dictate to others needs and get in the room what is needed

- During surgery, able to call out for instruments and needs before the doctor does

- Have eyes in the field at all times. Cannot reach out or over to get something when needed in the mouth. Cannot take eyes out of the field of work, must make sure all is retracted and clear for doctor, also makes sure nothing falls back to the throat. Keeps patient safe and comfortable

- Direct patient’s head

- Call for 2nd assistant when needed

- Make sure the doctor takes care of all we are scheduled to do, watch to make sure he does not make any errors and look to see if he missed anything, including needing more sutures.

- All progress notes and transactions put in

- Making sure the logging of any implants, drills, drugs or materials is done

- Making sure x-rays are taken after sx.

- Making sure the correct next visit is scheduled

## Periodontal Surgery or Crown Lengthening

*Complete sx. Set up checksheet if within the 3 month trial period for employees

- 2 Big Blu papers

- Perio kit

- Water bath and dishes

- Cavitron

- Curette pack  (1/2, 11/12, 13/14, 15/16)

- Extra Surgical pack of instruments (sugarman, younger goods, back action, 2 perio chisels, 2 perio files)

- High speed

- Small Suction

- Skinny white suction

- Big Blue suction

- Retractor (Betty and Minnesota)

- Suture holder

- Mouth prop

- Syringes

- Standard bur block

- Coats

- Glasses

- Four 15c blades

- Two 12b blades

- Sutures (4-0 Chromic long --ask doctor for others)

- Monoject

- C- Sponge

- Sterile Water

- Septocaine and blue needles

- Have two lidocaine 1:50 carpules

- Have one marcaine

- Sterile Gauze

- Sterile sleeves (5” long)

- Sx.Gloves, hat, mask out for doctor

- Pt. hat and bib

- If Desensitizing: Hibiclens syringe, Vista syringe with new tips and super seal w/ dappen dish and micro brush. This is to be used as follows:

- Isolate and air dry area

- Apply Hibiclens for 40 sec. twice in a row, rinse with sterile water

- Apply Vista for 40 sec. twice in a row, rinse with sterile water

- Apply Super seal for 40 seconds 3 times in a row, rinse with sterile water

## Tissue grafting

*Complete sx. Set up checksheet if within the 3 month trial period for employees

- 2 Big Blue papers

- Perio kit

- Water bath and dishes

- Cavitron

- Curette pack  (1/2, 11/12, 13/14, 15/16)

- Extra Surgical pack of instruments(sugarman, younger goods, back action, 2 perio chisels, 2 perio files)

- PRF express Kit

- High speed

- Small Suction

- Retractor (Betty and Minnesota)

- Suture holder

- Mouth prop

- Syringes

- Standard bur block

- Coats

- Glasses

- Four 15c blades

- Two 12b blades

- Sutures (4-0 Chromic long and short, 4-0 Vicryl long and short, 6-0 Vicryl)

- Monoject

- C- Sponge

- Sterile Water

- Septocaine and blue needles

- Have two lidocaine 1:50 carpules

- Have one marcaine

- Sterile Gauze

- Sx.Gloves, hat, mask out for doctor

- Pt. hat and bib

- Citric Acid, cotton pellets, micro brush, another dappen dish

- Extra pyrex dish for saline (if appropriate for graft)

- Tunnelling

- Blue blade laid out but unopened

- Sterile sleeves (5” long)

# Pinhole Surgical Procedure

## Set up:

*Complete sx. Set up checksheet if within the 3 month trial period for employees

- Water bath

- Perio Kit

- Cavitron

- High speed

- Small suction

- Suture holder

- Retractors

- 2 syringes

- Mouth prop

- Extra surgical pack of Instruments

- Citric Acid

- Pinhole Instruments

- Standard Bur Block

- Glasses

- Coats

- Cotton pellets

- Syringe with Chlorhexidine

- 3cc Syringe

- Isodry

- 2 Big Blue papers

- One 15c blade

- Sutures (6-0 Vicryl and any other doctor prefers)

- Monoject

- C- Sponge

- Sterile Water

- Saline Water (only if PRF can’t be drawn)

- Septocaine and blue needles

- Have two lidocaine 1:50 carpules

- Have 5 Marcaine carpules

- Sterile Gauze

- Sx.Gloves, hat, mask out for doctor

- Pt. hat and bib

- Large blue suction (next to saliva ejector)

- Saliva ejector

- Topical with qtips

- Micro Brush applicators

- 16g. needle

- PRF Express Kit

Procedure Sequence:

- Prep root with a round or diamond bur to remove caries and fillings and flatten convexity of the root

- Subgingival scaling with ultrasonic scaler or hand scaler

- Cleanse roots with Citric Acid and cotton pellets

- Rinse with sterile water

- Rinse with chlorhexidine

- Rinse with saline water

- Isolate and begin

- Use 16 g. needle for pinhole

When on upper working to the left use instrument A

When on upper working to the right use instrument B

6 Strips = 1 Biogide

Biogides--Upper: 40x50 (2) and 25x25 (1) Lower: 40x50 (3)

When working on lower to left use instrument B

When working on lower to right use instrument A

# Implant Surgery

## Setup

*Complete sx. Set up checksheet if within the 3 month trial period for employees

- 2Big blue papers

- Perio kit (Ext. Kit if extractions being done throughout sx.)

- Water bath and dishes

- Implant motor hose

- High speed

- Implant handpiece

- Small Suction

- Retractor (Betty and Minnesota)

- Suture holder

- Syringes

- Implant Kit (Sx. guide kit if sx. guide is being used)

- 3cc syringe

- Standard bur block

- Coats

- Glasses

- Three 15c blades

- Two 12b blades

- Sutures (4-0 Chromic long)

- Monoject

- C- Sponge

- Sterile Water

- Sterile Suction hose

- Septocaine and blue needles

- Have two lidocaine 1:50 carpules

- Sterile Gauze

- Sx.Gloves, hat, mask out for doctor

- Pt. hat and bib

- Have implants and healing abutments in room

- Topical

- PRF Express Kit

- Sterile Plastic tubing for handpieces and suction hoses (5” long)

- Big Blue suction

## After procedure:

- Take CT of implant

- Write up clinical notes include the size, and note if a healing abutment was used.

See example below:

CMZ

Sx. done today:

Imp. #13-(3.8x10.5) placed 3 healing abutment

BG: Mineross +PRF and Biogide

Waiting 2 months

See MVF dictation for detailed sx. note

NV: POT 1 in 2 weeks

- Remember to note how long we are waiting to uncover or osstell

- Complete transactions for entire procedure

- Open the tooth chart and create a new clinical note

## Logging IMPLANTS

- On the right side find “MVF Quick text”- Double click to insert

- Fill out the quick text information

- Highlight the note- Right click to cut

- Go to the “treatment Planner” (the green chair)

10. Go to “Supporting Information” on lower left

11.  Above that, click on “select Consent Form:” Select “Implant Log” and click “Add”

12.  Select “Additional Notes”  and then right click and paste the Implant Log into the white screen.

13. Click “ok”

14. Click “Save” (nothing will happen) and then “close”

15. Now go to the “Document Center”

16. Select the Form titled “Implant Log”

17. Click on “Modify Document” (top tool bar next to trash can)

18. Click on “Additional Notes”

19.  You may have to right click and paste the Implant Log again here.

*IF PATIENT ALREADY HAS IMPLANTS WE PLACED IN THIS LOG,  ALL YOU WILL HAVE TO DO IS THE ABOVE STEPS #15-19

Surgical Guide Kit

Solid Bite Surgery

This can be placing a full arch of implants (4 to 6) on an edentulous case and fixing in a prosthesis the same day or Extracting all teeth, placing a full arch of implants (4 to 6) and fixing in a prosthesis the same day.

The following is the set up for Ext., Alveo, Implants and Multi-Unit Abutment placement for a temp. fixed prosthesis:

**Complete sx. Set up sheet, and Cheat Sheet and sheet used for SB procedures**

- Surgical Table cover

- Plastic Bags for all 3 monitors and trash(this gets taped up between 2 counter tops behind MVF)

- Red Bag

- Ext. kit---be sure all things are inside

- Appropriate forceps and misch (tighten all forceps)-be sure all forceps are in and look good

- Proximators

- Water bath and dishes

- Implant motor hose

- Alveo burs

- Isodry

- High speed

- Implant handpiece

- Nose cone slow speed top

- Small Sx. Suction

- Sx. HVE suction

- Saliva ejector

- Retractors (Photo, Betty and Minnesota and large gold handle if on lower, red stripe for upper)

- Suture holder

- Mouth prop

- Lg. Bite block

- Syringes x 2

- Implant Kit- be sure all things are there (Lindemen, screw driver x 2, short and long straight latch and all other burs with a NEW pilot)

- BH Sx. guide kit

- Versah burs (if MVF says)

- Sterile plastic tubing for each handpiece and suction hose

- Piezo set up with bur blocks and water. Water bag should have a red warning on it. (if MVF says)(check to see if tips are good)

- 3cc syringe

- Standard bur block (be sure everything is inside)extra black diamonds, metal cutting and XXL’s)

- Coats (2 Med, 1 Lg)

- Glasses for pt. and asst

- Six 15c blades

- Three 12b blades

- Sutures (4-0 Chromic long x 3)

- 10cc syringes x 3 with blunt ended needles

- C- Sponge x 3

- Sterile Water (with extra to the side-not in cabinet)

- Sterile Suction hose

- Septocaine and blue needles

- Have two lidocaine 1:50 carpules

- Have 10 Marcaine

- Sterile Gauze Lg x 3

- Sx.Gloves, hat, mask out for doctor- 7 ½ for MVF and 8 for LNS

- Pt. hat and bib

- Have implants and multi units in room

- Topical

- Salvin torque wrench set with long and short drivers

- Multi-unit tools and blue carriers

- FMX, CT scan and Invivo up

- Large blue paper covering the side tray

- PRF Express Kit

- Electrosurge set to the side

If doctor is Preparing for conversion and doing final impression the same day:

- Blue Bite Registration material

- Light Body Material

- Heavy Body material

- 2 Guns

- 6 tips

- Q-tips x 3 with vaseline

- Long piece of floss

- Triad Gel

- Curing Light

- EZ Pick up or Quick up with 2 tips

- Patient parts (impression copings, chimneys, and analogs)

- Cotton pellets

Double Arch SB Sequence:

- IV sedation-Kiffel-see anesthesia record

- PRF drawn - left

- Ext. MX

- ALveo MX

- Implants Placed

- MUA chosen and caps placed. Blue mousse bite reg taken inside Mx denture

- Lab approved angulation and MUA were torqued.

- BG: Autogenous bone by bone mill and PRF membranes placed

- Sutured MX

- ·         Impressions to be done at this point if the doctor decides

- ·         Ext. MD

- ·         Alveo done

- ·         Implants placed

- ·         MUA chosen and caps placed. Blue mousse bite reg taken inside Mx denture

- ·         Lab approved angulation and MUA were torqued.

- ·         BG: Autogenous bone by bone mill and PRF membranes placed

- ·         Sutured MX

- -          If doctor decides, impressions to be done at this time:

- ·         Numb MX if necessary and MD with Marcaine

- ·         Wait to receive denture from lab

- ·         CT scan

- ·         Then screw prosthesis in and check bite

- ·         Access holes plugged with cotton pellets and light body material

- ·         see MVF notes for results of conversion completion

Single Arch SB Sequence:

·         IV sedation-Kiffel-see anesthesia record

·         PRF drawn - left

·         Ext. MX

·         Alveo MX

·         Implants Placed

·         MUA chosen and caps placed. Blue mousse bite reg taken inside Mx denture

·         Lab approved angulation and MUA were torqued.

·         BG: Autogenous bone by bone mill and PRF membranes placed

·         Sutured MX

·         If doctor decides, impressions to be done at this time:

·         MUA caps replaced

·         Wait to receive denture from lab

·         CT scan

·         Then screw prosthesis in and check bite

·         Access holes were plugged with cotton pellets and light body material

·         see MVF notes for results of conversion completion

# NSEQUENCE

Surgery Process: *Set up should be the same as the above “Solid Bite Surgery”

1.       Numb

2.       Place start bite

3.       Ext. teeth as minimally invasive as possible

4.       Flap to tuberosity and elevate to suture buccal flap to lip

5.       Suture palatal flap to palate

6.       Seat Foundation Guide (always adjust mouth to fit guide not vice versa)

7.       Place monostrut (found in foundation guide container)

8.       Place plastic pins

9.       Have pt. close

10.   Fixation drills

11.   Place fixation pins (these will stay in until ready to suture)

12.   Verify with lower teeth

13.   Take out plastic pins

14.   Remove monostrut

15.   Reduce bone with Foundation Guide (alveo)

16.   Place Surgical Guide on top of Foundation Guide and get the pins for this in place as well

17.   Use depth stop handle and follow the SP#  according to the plan sent with nSequence.

18.   Use placement tool on guide kit to place implant. When placing implant, the dot should line up with the blue line on guide.

19.   Remove Surgical Guide and keep Foundation Guide as is

20.   Place Angle Abutment Guide

21.   Place MUA carrier pin with blue line to line up on angle abutment guide

22.   Remove Angle Abutment Guide

23.   Torque MUA ONLY when MVF says that bridge is fitting passively and ready to lute

24.   Place Chimneys

25.   Place Gasket (place Vaseline on gasket being sure NOT to touch chimneys)

26.   Place red pins inside chimneys

27.   Paint Vaseline on Bridge

28.   Place MX or MD  Bridge

29.   Place Prosthetic Bite

30.   Have pt. close

31.   Lute using Dualine, EZ pick up, Quick up (Light Cure)

32.   Remove Prosthetic Bite and Bridge

33.   Remove gasket

34.   Place other chimneys

35.   Place Gasket (place Vaseline on gasket being sure NOT to touch chimneys)

36.   Place red pins inside chimneys

37.   Place MX or MD Clear Bridge and place Vaseline on this

38.   Place Prosthetic Bite

39.   Lute using Dualine, EZ pick up, Quick up

40.   Remove bite, bridge, and gasket

41.   Place MUA caps

42.   Remove the Foundation Guide

43.   Bone graft and suture (someone to polish the Bridge during this time)

44.   After luteing with Dualine, EZ Pickup, Quick Up through outer access holes:

45.   Set with curing light for 2 minutes to secure prosthesis to chimneys

46.   Remove prosthesis with chimneys attached

47.   Place red plastic pieces in chimneys

48.   Fill in prosthesis accordingly around the chimneys with Dualine, EZ Pickup, Quick Up and flowable composite if necessary or touching the “crown” surface.

49.   Cure with curing light for 2 minutes

50.   Remove red plugs

51.   Smooth with acrylic bur and polish with stone burs around the tops of the chimneys and access holes

52.   Buff with pumice, water, and fabric wheel until completely smooth.

53.   Wash, Air dry and Paint Palaseal then cure in oven for 4 mins.

54.   Remove MUA caps

55.   Place Bridge

When SB sx. is complete:

- A complete log of Implants, Imp. torque, MUA, MUA torque should have been filled out on a separate sheet

-The above sheet should be placed on MVF desk to dictate

- All implants should be logged in transactions and transactions completed in treatment plan. All materials and drills to be logged in Surgical Log book as well.

- Prior to CT scan approval, all surgical instruments can be taken except for Imp. Kit, Imp. motor, Imp. handpiece and PRF

When writing up the progress notes for a Solid Bite sx., be sure you double click the “SB” quick text to add and copy and paste ALL the NV.

# Bone Expansion

Bone expansion is done in an area where the bone is very thin, but the doctor feels that if expanded, an implant could be placed in the expanded spot on the same day. The bone expansion kit is kept in the sterilization lab and on the shelves over the statim.

How to use:

Give doctor the starter drill and work your way up with the drills

Let the doctor choose a long or short expansion extender

Begin passing the extenders. (USE a C-Sponge, these tend to fall out easily) All the extenders are named A1, B1, C1, D1, E1, E2, F1 etc. they’re marked on the drill itself.

Once expansion is achieved, switch to the implant kit.

Trichloracetic Acid Routine

Materials needed:

-Glycerine

-Several rolled up cotton pieces

-Trichloracetic acid w/ glass dappen dish

-Radiosurge

-Sterile Water

-Curettes and or Cavitron

-Etch

-Geristore

-Standard Bur block

-Curing Light

-High Speed handpiece

Procedure Steps:

Place Glycerine on lips, gum tissue, and surrounding teeth from site

Place Trichloracetic acid in glass dappen dish

Dip rolled cotton pieces in acid and dab off excess

Place cotton pieces on site for 2 minutes

Remove cotton pieces at 2 minute mark

*Radiosurge may be used at this time on the surrounding tissue

Irrigate several times with sterile water

Curettes and /or cavitron to be used

Rinse with sterile water

Air dry, place etch, rinse with water and dry completely

Apply Geristore to the site and light cure

Adjust and smooth off Geristore

### Returning Failed Implants

1.   Place failed implant in sterile bag.

2.    Write patient’s name on outside of bag and date.

3.    Sterilize the bag

4.    Place sterile implant in returns box.

### Sending Implant for Credit

- call the implant company

- Give sales operator the name of patient, size, lot, and reference number on failed implant.

- Tag is posted in patients chart.

- Company will fax you over a form and send you a replacement implant.

- Fill out form.

- Make a copy of form.

- Place copy in proper scan folder.

- Place sterile implant and copy of paper work in a box or mailing folder.

- Call ups or fed ex. (You can send USPS) and send package.

- Write the cpu # in the ups log book with name and address of return.

## Discharging Patients after a sx. Procedure

## If placing implants: Be sure that a CT was taken and approved by MVF prior to discharge or cleaning up room.

## Be sure that Stella Life Gel was applied to the affected area.

- Be sure Marcaine was given (if applicable).

- Be sure that any driver was called (if applicable)

- Ask doctor in the room if a post op visit is necessary and if it is needed to have a doctor or not. If so print the NV card. If not, be sure that a note is given to the front staff to cancel this appt. (During FA, they are making a post op visit, but not telling the pt. It is up to the sx. staff to ask MVF in the room and cx or keep the appt.)

## Be sure that all instructions are told to pt. and are specific for this procedure that was done.

- Be sure that gauze pack, post-op instructions, Ice pack and NV card are all given to pt. or driver.

- Pt. should be escorted out the back door and directed or walked to the parking lot if there are other patients in the waiting room.

## Clean Up After Surgeries

*Complete sx. Clean up checksheet if within the 3 month trial period for employees

- Take all blankets, jackets and washable head covers to the staff bathroom laundry basket

- Dispose of all sharps and hazardous waste

- Return all unused supplies to their designated area (implants, materials) and be sure to spray all that was in direct contact with spray or blood. Spray with disinfectant and lay to the side to dry

- Wash all safety glasses with soap and water and lay to the side

- Remove all barriers

- Take all instruments to the lab

- Spray entire room with disinfectant:

- Patient, assistant and doctor chairs

- All counters in operatory

- All cabinet facings (ALL)

- All units, all handpiece bases and cords, all hoses and the holders they sit in

- All keyboards, lights. Xray units, light systems, head light handles, anything that would be touched!

- Leave disinfectant on all while you continue to put things away and clean up

- Take out all garbage

- Get Purevac bucket and submerge any suctions used (to the assistant’s discretion).

Also be sure to clear out saliva ejector with Q-tip to clear out blood clots.

- Once bucket is empty, take a disinfectant wipe and wipe all hoses and cords clean. Wipe the entire line. Be sure the hose and cord holders are disinfected and clean before returning.

- Wipe all hand piece bases and cords with a disinfectant wipe (these should have been sprayed prior as well)

- Sweep floors, sweep out pedals, sweep under patient chairs and under counters and unit bases free of debris and acrylic dust. Mop with swiffer (sweep and mop up to assistant’s discretion)

- Check floors, walls, chairs, headlights and units for blood splatters and messes

- Check all units, machines, holders

- Place new barriers on all

- Think of all the areas and things that an assistant would touch during that visit, make sure these things are wiped and disinfected for the safety of all (door knobs, keyboards, mouse, light system, head light handles, xray units, cabinet and drawers handles, pens)

- Clear out of all patient xrays and notes

## Upper Extraction

- Extraction Kit (orange/yellow long kit)

- Orange striped forceps (Upper) and Misch associated with the correct location (left, right, anterior)

- Proximators

- Implant motor (If implant being placed in conjunction with extraction)

- High speed

- Cone top slow speed (if there is any alveo or an adjustment necessary after sx)

- Implant handpiece(If implant being placed in conjunction with extraction)

- Small Suction

- Retractor (Betty and Minnesota)

- Suture holder

- Mouth prop

- Syringes

- Standard bur block

- Coats

- Glasses

- 2 Big blue papers

- Three 15c blades

- Two 12b blades

- Sutures (4-0 Chromic long and short)

- Monoject

- C- Sponge

- Sterile Water

- Septocaine and blue needles

- Have two lidocaine 1:50 carpules

- Sterile Gauze

- Sx.Gloves, hat, mask out for doctor

- Pt. hat and bib

- Large blue suction (next to saliva ejector)

- Skinny white suction

- Saliva ejector

- Topical with qtips

- Isodry (if necessary)

- PRF Express Kit (if necessary)

# Lower Extraction

-Extraction Kit (orange/yellow long kit)

-Orange striped forceps (Lower) and Misch associated with the correct location

- Proximators

- Implant motor (If implant being placed in conjunction with extraction)

- High speed

- Cone top slow speed (if there is any alveo or an adjustment necessary after sx)

- Implant handpiece(If implant being placed in conjunction with extraction)

- Small Suction

- Retractor (Betty and Minnesota)

- Suture holder

- Mouth prop

- Syringes

- Standard bur block

- Coats

- Glasses

- 2 Big blue papers

- Three 15c blades

- Two 12b blades

- Sutures (4-0 Chromic long and short)

- Monoject

- C- Sponge

- Sterile Water

- Septocaine and blue needles

- Have two lidocaine 1:50 carpules

- Sterile Gauze

- Sx.Gloves, hat, mask out for doctor

- Pt. hat and bib

- Large blue suction (next to saliva ejector)

- Skinny white suction

- Saliva ejector

- Topical with qtips

- Isodry (if necessary)

- PRF Express Kit (if necessary)

## Uncovering

### Set Up for a full arch of Implants: (If uncovering single implants, only the BOLD are necessary to have open-per MVF and LNS 10/28/17)

- Uncovering cassette

- Prosthetic Kit or Single Driver

- Bone Profilers (do not need to have out, but know where they are)

- Latch grip handpiece (do not need to open, but have in room)

- Posterior Driver (do not need to open, but have out)

- Syringes x 2

- Retractor

- Suture Holder

- Gauze

- water bath

- Sx. Table Cover (if doing a full arch, if not, large blue paper-per LNS 08/2013)

- Dexis with appropriate rin

- Suction hose or Skinny white suction tip

- High speed Hand piece (do not need to open, but have in room)

- Bur Block ( do not need to open, but have in room)

- Chlorhexidine rinse in cup

- Assistant gloves, cap, mask-(non-sterile are ok for single implant cases)

- Doctor gloves, cap, mask(non-sterile are ok for single implant cases)

- Pt. hat, glasses, bib (non sx if single implant)

- 2 Q-tips with topical

- Appropriate anesthetic and needles

- #15 Blades x 1

- #12 Blades x 1

- 4-0 Chromic suture

- Monoject

- Osstell Machine, peg and wrench (use sleeve to cover cord and buttons)

- Dr. Might want to use electrosurge

- Appropriate healing abutments out on counter

- Appropriate Tissue Punch

- Dappen dish with a little hydrogen peroxide and micro brush

### Steps to uncovering:

- Complete sx. Set up checksheet

- Have sx. Table ready to go

- Seat patient and place sx. Bib and hat on them

- Take PA of implant. (Print PA and place in envelope with parts to send to G.D.-if applicable)

- If uncovering a full arch, take a new CT scan

- Hook up BP and Pulse oximeter

- Have patient rinse with chlorhexidine

- Once doctor enters room, place topical in the area where the implant is and lay patient back.

- Have a short needle with correct anesthetic loaded and hand doctor.

- Hand doctor #15 blade for initial incision

- Use a tissue punch if there is enough keratinized tissue

- Hand doctor screwdriver to unscrew cover screw

- Ask doctor what size healing abutment will be needed-call for asst. if necessary.

- Hand doctor osstell wrench and peg. Once this is tested, make note of reading.

- Take osstell wrench and peg and have second assistant get the correct healing abutment.

- Use hydrogen peroxide around implant

- Load healing abutment on screw driver and give to doctor.

- Doctor may need to suture. If so, have suture loaded and in hand for doctor. (4-0 Chromic)

- Give instructions to patient.

- Write up progress notes, being careful to note if you sent parts with patient to G.D. or to dictation, note the osstell reading and the size healing abutment you used and size of implant (use Quick Text )

- Put in transactions.

- Prepare NV sheet for POT4 after final crown if outside GD is doing abutment and crown. Otherwise, if stitches are involved, wait 2 weeks to schedule Itero for custom abutment and crown.

- Schedule appropriately

## Osstell Check

Set up for Osstell Check:

- Osstell machine, peg and wrench

- Prosthetic Kit or just a screw driver

- C-sponge

- Dexis for PA if necessary

- Healing Abutment if cover screw is there

- 2 Large blue papers

Procedure Steps:

- Take a PA

- Place C-sponge

- Unscrew the healing abutment

- Place osstell peg w/ wrench or hemostat

- Place plastic sleeve wrap over wand and barrier tape over osstell machine

- Place wand to the top of the peg and wait for a reading ( you should hear a clicking sound if it is in the right position) COVER SUCTIONS.

- If the reading is at 55 and above, implant is ready for restorative work to be done.

- If the reading is under 55, make sure that the screw is properly placed and tightened all the way. There may be a time that soft tissue is involved and will require a doctor check.

- Replace the healing abutment

- Remove the C-sponge

- Let the doctor know the reading to make sure it is ok to send the appropriate parts to General Dentist or schedule NV for Itero if we are doing the custom abutment

- Note everything in the progress notes and send parts to dictation

* NOTE: Type 27 Osstell peg is for BioHorizons internal implant for a 3.8, 4.6 or 5.7 implant. If implant is a 3.0, use Type 26. If another implant is placed, you need to find out which type of peg. www.osstell.com*

*NOTE: Per MVF and LNS, ok to scrub pegs with a brush and wipe down with alcohol wipe prior to going in autoclave

# Quick Text FOR UNCOVERINGS AND OSSTELLS  Being scheduled

If NV is to be an uncovering or osstell, the following Quick Text (found in Clinical Notes to the right) should be selected and filled out.

Quick Text is named “Uncovering:”

- Implant # and sizes?

- Are any of the implants exposed? If yes, which ones?

- Does pt. need an adjustment at uncovering?

- Who is GD? If MES, can we do Itero the same day?

- If MES is not GD, give note to have parts pulled for GD.

- Has pt. paid for next phase of tx. (cust. abut. and crowns)? If not, set up FA prior to uncovering.

- Did pt. have IV for implant sx or require a medical clearance? If so, this will need to be reviewed by doctor and decision made.

## Electrosurge/Radiosurge set up

*This is a product of Ellman and they should be contacted at www.ellman.com  for any repair/replacement*

- Set out machine and plug into electrical outlet. Place pedal on the floor.

- Plug in the black cord into the black slot on the machine and the green cord into the green slot into the machine.

- Find the appropriate electrode tip the doctor wants

- Twist the neck of the handpiece and place electrode into the slot. Twist the neck to close (this will tighten around the electrode shaft).

- Place the white “paddle” under the shoulder of the patient.

- The top switch on the machine should be turned to where it’s pointing to “CUT 1”

- The middle switch/dial should be turned to about 4. (Doctor to adjust as necessary.)

- After numbing the patient, the assistant should be ready with gauze in one hand and a high volume PLASTIC suction in the other hand to collect the “smoke” that rises. The smell is terrible, so keep close to the “smoke” to be sure it’s caught before it reaches the patient’s nose.

- When done, the handpiece neck is turned and the electrode removed. This is not to be submerged in water, but can be wiped down and placed in autoclave.

## Piezo Setup

- Get out bag of sterile water in basket from stock room marked Piezo. (500ml sterile water bag IT HAS A RED WARNING LABEL ON IT)

- Get out Piezo basket in room 6 behind the pt. chair on shelves under counter

- Get out hand piece set up/burs/crown/lines The pedal is wireless.  The only time we will use the cable is when the battery in the pedal is dead

- Take the hand piece and hook it up to the machine

- Put the sterile water on the hook (sterile water bag will have a RED WARNING LABEL. This bag is only used for the piezo and should be set apart from the others)

- Hook up all the lines before connecting the line to the sterile water

- Make sure the line is placed correctly in the pump (line runs from back of machine to front)

- Turn machine on in the back

- The Piezo will load and you will have to read through directions and hit next until it asks you to confirm that the pedal/handpiece  is working by pressing the pedal.

- The machine will need to be flushed for about 1-2 mins before procedure.  Hit the flush button and hold hand piece over trash can or sink.  If you hit the pedal it will go to a separate screen.  This will flush itself.

- Put the bur in the hand piece by twisting it in by hand and then use crown to tighten it two full spins around.

- There are two setting we use on the screen: Press the “Bone Cut” and keep the speed at 20-25, “Root” The highest the speed will go is 18.  Keep the water at 3-4 depending on what doctor wants.  Always tell him what the speed is and the amount of water.

- The machine is touch screen, and the knob controls the speed and the water amount

## Auto Tome

- Make sure the Auto Tome handpiece is plugged into the implant motor (or slow speed line) and power is on

- Make sure setting is to 15,000-30,000 RPM 1:1.

- Turn dial at the top of the handpiece neck to the “unlock” position  and place the appropriate bur inside. Turn dial at the top of the neck to the “lock” position.

- Test handpiece to be sure it is working

- Hand piece and bur/bur block can be sterilized

Sonic Weld Procedure

Sonic Weld Unit

Sonic Weld Tips

Sonic Weld Vertical Foils

Sonic Weld Horizontal Foils

Sonic Weld Pins (company calls these the implants)

·         Contact  http://www.klsmartin.com/produkte/dental/sonicweld-rxR-dental/das-verfahren/

·         Representative is Rocky w/ KLS Martin

Ceraroot Implant Placement (Ceramic Implant)

Set up:

- Use the “Implant Set Up” hat to begin

- The drills in this kit are drills for parallel walled implants

- If placing the Implant size 14, this will be done using the tap technique. The mallet and the “inserting tool” are used for this and are found in the bottom of the kit.

- For info, ordering, drill sequence etc. go to: http://www.ceraroot.com/professionals/products/surgical-parts/

You can contact Xavi Oliva at  (800) 485-1613

IV Sedations

## IV by Sheldon Staff

### Set Up:

- Get out surgery log book

- Place Blue paper next to sink and if in room 6, place on counter under TV

- Place following items on blue paper:

- (2) 3cc syringes

- (2) alcohol wipes

- Several pieces of gauze

- (1) piece of clear site tape

- (2) 1 inch pieces of tape

- (2) 22 gauge catheters

- (1) extension set

- Basket of drugs from drug cabinet

- Tourniquet and squeeze ball

- Set out 1 bottle of Versed (Midazolam) and wipe top off with alcohol wipe

- Have Fentanyl, reversal drugs, and Benadryl in basket

- EKG Machine

- EKG Leads

- Coe-ease

- O2 Cannula w/ EKG line attachment

- Set up IV bag/line:

  - Place long metal rod on ceiling hook

  - Hang IV bag of 5% Dextrose w/ sodium chloride  or 5%Dextrose w/ Lactated Ringers from metal rod

  - Remove blue plug from bag or white plug if using D5LR and insert IV line administration

  - Fill the drip chamber ½ way by squeezing

  - Attach extension to end of IV line

  - Hang line into sink and let solution flow through line until there are no bubbles in line

- Emergency tackle box, ambu bag and laryngoscope box all in room

### Prep Patient:

- IV assistant reviews IV Sedation log sheet located in the surgical log book

- Take initial BP and record

- Review MH

- Give pt. squeeze ball and place tourniquet

- Find a vein

- Use 22 g. catheter, advance and attach extension that’s connected to IV line

- Remove tourniquet

- Open drip gauge to get a drip through the line

- Grab a 2x2 and apply pressure just above catheter

- Place clear site tape over hook up where the IV line attaches to catheter

- Use the (2) 1 inch pieces of tape to secure the line to the surrounding area

- Assistant alerts the doctor to say hello to pt. and reviews stats and MH with him

- Assistant administers drugs per doctor's orders

- Pt. vitals to be recorded every 30 minutes

- Pt. never to be left alone

- Be sure male to female ratio is correct at all times (ie: if there is an instance where a female asst. is left with a male pt., there needs to be another female called in. If a female asst. has to be alone with a male pt, the door should be open and the asst. in the doorway)

- Drug lock

### Discharge:

- Shut off solution by turning off drip gauge

- Carefully remove tape

- Place gauze and apply pressure over catheter and remove catheter

- Place bandaid or bandage

- All post op instructions to be given to the driver and discharge info to be noted on the Sedation log.

New Policy for Sedations 09/05/2015

We will do either no sedation, Twilight Relaxation Sedation or Anesthesiologist Sedation.

You must get the Doctor to authorize which one they can do in our office if sedation is requested and have Dr. sign off the Medical History stating that this is ok. If a procedure is over three hours we recommend Anesthesiologist Sedation for the comfort of the patient and the Doctor to work efficiently, However, you must have the Doctor ok and sign off the Medical History for this.

Pre-operative Instructions for Sedation

If a long surgical period is anticipated, we may recommend or you may prefer sedation in addition to local anesthesia.  Sedation provides you with additional relaxation and places you in a “twilight sleep.”  Time passes more quickly under sedation, and most patients do not even remember the surgical procedure.

The following recommendations will help to ensure a safe and comfortable sedation.

- Arrangements should be made to have someone drive you to and from the office and stay with you when you get home.

- No solid food should be eaten within eight hours of the procedure.

- Clear liquids (apple juice, tea, black coffee, clear broth, Jell-O, water) may be eaten in reasonable amounts until three hours before treatment.  No other food should be eaten.

- Restrictive undergarments should not be worn.  Please wear loose, non-restrictive clothing with short sleeves.

- After surgery, please plan to go home and rest.

- Patient’s ride should be aware of the quiet environment recommended. ( see “Silence is Golden” in consent bucket)

- If a patient is diabetic then advise patient not to take his or her diabetic medicine the morning of the IV sedation surgery.

Top ten emergencies in dental office:

- #10 Cardiac arrest (before or after treatment)

- #9 Insulin shock (hypoglycemia for type I or type II diabetics)

- #8 Epinephrine reaction (caused by rapid uptake of epinephrine in Local Anesthetics; if during a sedation, may notice monitors show an elevated heart rate; try to limit it, especially in cardiac patients)

- #7 Hyperventilation

- #6 Asthmatic attack (sedation may relax these patients more; differentiate between the mild, moderate, and severe asthmatic patient prior to sedation; as what meds- just inhaler, or also steroids, hospitalizations)

- #5 Seizures (sedation raises the seizure threshold, especially with benzos; dental procedures can trigger seizures by Local Anesthetics use, especially in pediatric patients – know how much kid’s lean weight is)

- #4 Postural hypotension (especially geriatric patients, and those patients on hypertensive meds)

- #3 Angina pectoris (chest pain, but not a true heart attack)

- #2 Mild allergic reaction (clear quickly with Benadryl, true anaphylaxis is rare)

- #1 Syncope (fainting = most common in ages 15-35yo)

Time of occurrence of systemic complications:

- 1.5 % just before treatment

- 54.9% during/after Local Anesthetics (so monitor patient, and do not leave patient unattended)

- 22.9% during treatment

- 15.2 after treatment

- 5.5% after leaving office

ASA classification:

- ASA I: normal healthy patient

- ASA II: mild systemic disease

- ASA III: significant systemic disease

- ASA IV: significant disease, constant threat to life

- ASA V: moribund patient

- ASA VI: organ donor patient

Blood pressure	Dental treatment

Normal 	<120/<80 		No modification

Pre-HTN 	120-139/80-89 	No modification

Stage I 		140—159/90-99 	No modification, medical referral, notify patient

Stage II 	160-180/100-109 	Selective dental care, medical referral

Stage III 	180-209/110-119 	Emergent care

- BP levels fluctuate the most in the morning, so afternoon appointments are preferred.

- Anticipate orthostatic hypotension – slow movement from supine to standing

- Association between bleeding and hypertension is questionable and probably not an issue.

ORAL SEDATION

| Drug | Proprietary | Peak plasma level (hr) | Half life (hr) | Active metabolites | Availability (mg) | Dose (mg) |
| --- | --- | --- | --- | --- | --- | --- |
| Alprazolam | Xanax | 1-2 | 12-15 | No | 0.25, 0.5, 1 | 0.25-0.5 |
| Diazepam | Valium | 2 | 20-70 | Yes | 2, 5, 10 | 10 |
| Lorazepam* | Ativan | 2 | 12 | No | 0.5, 1, 2 | 2-4 |
| Oxazepam | Serax | 1-4 | 5.7-10.9 | No | 10, 15, 30 | 15-30 |
| Triazolam* | Halcion | 1.3 | 1.5-5.5 | No | 0.25, 0.5 | 0.25-0.5 |

*preferred oral medications 1 hour before conscious IV sedation

SEDATION PROTOCOLS

- Short procedures (about 3 hours):

  - Antibiotic pre-med oral or IV (for implant surgeries or surgeries that involve non-autogenous grafting) = 2grams of amoxicillin (oral) or ampicillin (IV or IM).

  - Start with 2mg of Versed.

  - Wait 3 minutes and give a 50mcg of fentanyl.

  - Titrate Versed (give 1mg) as needed (or at every 10min) to maintain sedation.

- Long procedures (more than 3 hours):

  - Antibiotic pre-med oral or IV (for implant surgeries or surgeries that involve non-autogenous grafting) = 2grams of amoxicillin (oral) or ampicillin (IV or IM).

  - Start with 50mg of diphenhydramine.

  - Wait about 3-5min and give 2mg of Versed.

  - If procedure is long, a 50mcg IV dose of Fentanyl 3min after the first Versed dose is optional (another option is to give 5mg of morphine INSTEAD of the first Versed dose – letter “c” above).

  - Wait at least 10 min and titrate Versed (give 1mg) as needed (or at every 10min) to maintain sedation.

IMPORTANT NOTES:

- MAXIMUM DOSES

  - VERSED = 10mg (prevents re-sedation when reversal is given)

  - MORPHINE = 8mg (decreases chances of PO2 inhibition)

  - FENTANYL = 50 mcg (short duration, rapid onset, 100x more potent than morphine)

- EMERGENCIES

  - If patient symptoms are indicative of over-sedation or adverse reaction to the sedative given, calling 911 and aborting the procedure are a must. Let the EMT team get to the office and do their work ASAP. No hesitation.

  - The only 2 emergencies that won’t require immediate 911 calling are fainting and seizures (both usually happen before treatment is rendered)

  - The doctor will recommend the following reversal protocols:

    - Flumazenil (Romazicon): Versed reversal

      - Give in 0.1-0.2mg increments (no more than 2 doses  before calling 911)

    - Naloxone: Fentanyl or Morphine reversal

      - Give 0.1-0.4mg increments (no more than 2 doses  before calling 911)

      - Beware of adverse reactions = pulmonary edema; HTN; cardiac arrest.

# Anesthesia Guidelines for Anesthesiologist

We provide all drugs and materials. The doctor will provide the Propofol pump if they see fit.

Regarding fee collection, he would greatly appreciate if our front office would collect fees at the point of service. Eleanor will reimburse.

The minimum is $3150.00 and up to 8 hours, $525 every hour after that.  The time starts when he is scheduled to be here with pt and ends when he leaves the pt.

His preferred time is around 8 and we are scheduling on Fridays.

Choose a date of surgery based on the scheduling template and let your patient know that we will call within 48 hours if the anesthesiologist needs to change the time or day, so tell them, don’t make hard arrangements yet.  The scheduling coordinator will email Dr. Kiffel  right away with the patient's name and surgery day/time along with any medical or emotional issues to consider. Dr. Kiffel will reply back by email with confirmation of the surgery date and any other information that he will need for surgery. Dr. Kiffel may ask for an EKG to be done and it must have the waves on the report. Most recent Medical History from their Family Dr. will be needed.  It is rare he will not be able to accommodate our sx day.  Once he confirmations, copy and paste this in the PC Sheet.  Inform the patient that Dr. Kiffel will be calling to go over his pre/post op, we do not need to go into any more detail than that, but just in case they ask, this is basically his pre/post op:* All females of child bearing age (which has not had a tubal ligation or hysterectomy) will be required to take a UPT day of surgery. This is to avoid a fetal anomaly. Birth control methods (oral, implanted, inserted or stickers) do not count for dismissal of this. * all regular medications can be taken with water am of sx, except diabetic meds because they are to NPO, if patients take daily pain meds or daily anxiety meds, this can and should be taken as well. Diabetic pumps are ok to keep on. *No eating after midnight, if the surgery is in the afternoon or late am, patients can have clear liquids only 6 hours before sx. *He goes over patient must have driver *No smoking 2 weeks (reduces coughing and lung secretions)

## Set up for Anesthesiologist sedations

- Front desk is to have signed anesthesia consent

- If necessary, Pregnancy test taken and results obtained before seating pt.

- Sticky note on consents of allergies and alerts

- Hat, mask

- Blue paper out:

  - Stethoscope

  - Tourniquet and stress ball

  - (2) 22 g. catheter

  - Several alcohol wipes

  - Stack of 2x2’s

  - Coe-Band

  - Clear site tape

  - (2) 1 inch pieces of tape

  - (1) IV line extension

  - (1) 60cc. syringe

  - (1) 10cc. syringe

  - (2) 3cc. syringes

- Nasal O2 Cannula that hooks to EKG

- EKG leads

- Restraint straps

- Drug basket containing reversal drugs, Benadryl, 2 vials of Fentanyl, Versed, and 1 box of Propofol

- Emergency drug Kit, trumpets, large/long suctions (all in cart/cabinet)

- Emergency Tackle Box, Ambu-Bag, and Laryngoscope box

**Assistant always to be with the Anesthesiologist**

How to work the EKG

TO change the settings for Sedation and back for non-sedation.

- Press Default-a yellow highlighted box will appear on the screen

- PRESS the gray knob to select what setting you would like monitored. For sedation,  select CO2, EKG, BP, SPO2 setting. For non sedation, select BP and SPO2 setting

- Turn knob to ACCEPT

- Push knob to ACTION REQUIRED

Emergency Protocol

- If there is an emergency in the office, alert the rest of the staff by using the walkie headpiece and calling code ER (then whatever room you’re needing assistance)

- ie: a patient begins to pass out during PRF in room 6, use your walkie and call code “ER6” then press the doorbell to alert all others that do not have a walkie

- NEVER LEAVE THE PATIENT for help. If you are unable to use your headset or get to the door bell for whatever reason, yell loud enough for someone to hear you. Between the headset, doorbells, and blue note, there is no reason why you should have to leave the room.

- If you are calling the code, remain calm and work quickly. Re-assure the pt. that we are helping them and continue keeping them talking if possible.

- If possible, place the pt. in a comfortable position (refer to Medical Emergency sheet for details), apply oxygen quickly, and begin BP and pulse oximeter.

- If responding to an emergency, hurry to sterilization room. Under the statim, the tackle box, laryngoscope, AED, and ambu bag are all located.

- Grab the above items as well as the O2 portable tank in sterilization

- Grab the EKG machine and bring to the emergency site. O2 cannulas have an EKG line that should be hooked up right away to the machine

- Begin setting up for IV

*All narcotics are in locked cabinet. Managers and doctors only to have code to the lock

Running an Air Driven Handpiece if Power Goes Out

Running the Generator

This is found in the “trash closet” outside under the stairs.

# 1.      Turn valve on top to “open”

# 2.      Turn black dial on the side to “on”

# 3.      Press switch on the side to “on”

# 4.      Pull the choke out all of the way

# 5.      Then pull the cord out towards you.

# 6.      Once the generator begins, slowly press the choke back in

# (NOTE: you may have to do #5 and #6 a couple times)

# Supplies Needed for Easy X-trac Kit

- Cartridge gun

- Blue Mousse

- Tips for bite reg.

- Left or right metal x-trac impression tray

- Latch handpiece with slow speed motor or implant hose motor

- X-trac drills (white, red, blue) and mount

## Procedure Steps:

- Have blue mousse with tip and cartridge ready for use.

- 28 mm and 36 mm screw ready for Dr to choose size.

- Bur block with white, red, and blue drills loaded.

- Ratchet ready for Dr to use.

- Dr will start out to use the white, red, then blue drill.

- Dr will then choose which screw to use.

- Fill all but the hole of the impression tray with blue bite registration material.

- Impression tray covers the area with screw sticking out of the hole.

- Dr seats tray in patient’s mouth.

- When set, doctor uses ratchet to remove tooth.

- Drills can be reused until dull.

- Drill will need to be scrubbed, individually bagged, and placed back in Easy X-trac kit.

# Mix & Place Coe-Packs

- Equal lengths of catalyst and base are placed on the mixing pad.

- Use a tongue depressor to mix until the colors are evenly blended.

- Scoop up the Coe-Pak on the tongue depressor and place in pan or cup of water

- Form a rope with Coe-Pak and place it over the surgical site gently pushing the material around and between the teeth making sure it is not on the occlusal surface.

- Instruct the patient to dab the perio pack with Vista bleach solution and not to brush and floss the  area.

# Coe-Soft

- Exam kit

- Coe-Soft kit (2 scoops Powder per arch; 8mL of coe-soft liquid per arch)

- Pink Disposable Cup to mix in and Tongue Depressor as mixer

- Slow Speed Hand Piece

- Acrylic Bur (hedgehog bur to ream out existing reline if any)

- Implant Acrylic Bur (to ream out Healing abutment areas on reline if implants are present)

- Flame

- Lighter

- Scissors

Coe-soft Process

1.   	Pt removes denture

2.   	Ream out old coe-soft

3.   	Try denture in pt mouth to determine how much coe-soft is needed

4.   	Mix 2 scoops of powder to 8ml of the liquid. Stir until the powder is all mixed in (no more than 30sec) then stop stirring and let it set up. Stirring occasionally only to check the consistency.

5.   	Pour mixture in denture

6.      Dry off arch with a gauze

7.   	Place denture in pt mouth and hold denture in place for 1 minute. Scoop out excess from the back (or have someone help with that).

8.   	If pt. has any opposing teeth, have pt bite down for an additional 2min

9.   	Remove denture

10.   Rinse and quickly trim any excess that may cause pt. to gag

11.   Place denture back in pt mouth for 5 min firmly

12.   Remove and trim all excess coe-soft from dentures

13.   Dismiss pt.

*Remind pt. that the liner should stay soft for up to 4 weeks and should not need to be re-done. (this is of course after primary swelling has gone down after any sx.) Explain to pt. that there may be sore spots that occur and can be adjusted.*

# Permasoft

# Soft Denture Liner for the use around implants with healing abutments

1.      Roughen the inside surface of the denture to make room for the soft liner.

2.      Clean and dry the areas where the soft liner is to be applied.

3.      Using the softest density, stir 8 ccs of powder into 4 ccs of liquid, stirring that very slowly until all of the powder is totally moistened. The mixture should turn sticky or honey-like.

4.      Immediately put the material into the totally dry denture. Ask the patient to lick their lips, and then place the denture in the mouth and tell the patient to close gently into occlusion for one minute.  While the patient is biting have them move their lips and cheeks around during that entire minute.

5.      Remove the denture from the mouth.

6.      Using a finger coated with the mixing liquid, use that to smooth the excess material on the buccal surface of the denture.

7.      Place the denture into a cup filled with hot water or the hot water bath in a temperature between 122-165 degrees Fahrenheit and leave that in there for 10 minutes.

8.      After the 10 minutes, chill the denture in cold water or ice water to temporarily harden the soft liner for easy trimming.

9.      Remove the excess material with sharp scissors or a scalpel and smooth with a large rubber point or finishing stone. Remove any material from the imprint of the healing abutment. You can use a molt or something equivalent along with a torch to heat the instrument.

10.   If desired, finish with a wet rag wheel and pumice to further smooth the liner surface, but do not high shine the soft liner.

11.   Dry the liner surface using air or warm air to remove all surface moisture and then apply a generous coat of Permasoft Sealer over the totally dry, soft liner and air dry for 2 minutes.

12.   Repeat that for a second coating.

13.   Do not place denture in patient’s mouth until the Permasoft Sealer is completely dry and rinsed.

Instructions for Patient

1.      Soak only 10 minutes in denture cleaner.

2.      Do not scrub the soft liner with a hard bristle brush.

PerioFlow

# Peri-Implantitis

- Set up as if a crown lengthening procedure

- Have Citric acid, saline, cotton pellets and Perioflow

Sequence is as follows:

- Raise the flap

- Cavitron to be used

- Perioflow to be used

- Citric Acid with cotton pellets (count the pellets aloud as they are placed) to sit for 2 minutes. Remove pellets (count aloud as they are removed)

- Irrigate with Saline

Pref Gel and Emdogain Sequence

·        Use citric Acid using microbrush and wait for one minute, then rinse with sterile water. (this is optional and up to the doctor to decide)

·        Use PrefGel with it’s own syringe and tip, wait for 2 minutes, then rinse with saline.

·        Place Emdogain on surface using it’s own syringe and tip.  DO NOT RINSE

# Discharge of Surgical Patients

After the procedure is done, you must do the following:

- Take any needed x-rays. (CT scans for implants must be taken and approved by the operating doctor before cleaning up anything related to the implant and before discharge-no need for LNS to check anymore -per LNS 5/1/17 before discharge)

- Go over post op instructions with patient or caregiver if patient is sedated.

- Give patient ice pack,  gauze pack, NV card, and post op instructions.

- All patients, discharge out the back door of room #3 or #6 to driver.

- Type progress notes in the computer and complete transactions.

# What the Cassettes Should Contain

Hygiene

- Mirror

- Probe

- Sickle scaler

- ½ curette

- 11/12 curette

- 13/14 curette

- 15/16 curette

- One set of files

- Prophy paste holder

Perio Kit

- Mirror

- Mirror

- Probe

- Probe/explorer

- Blade holder x3

- Goldman

- Molt

- Curved Molt

- Pritchard curette

- 4R/4L

- Prichard x 2

- Plugger/Plastic

- Packer

- Tissue Forceps x2

- Cotton pliers

- Hemostat

- Curved scissors

- Straight scissors

- Suction and hook

Extra surgical pack of Instruments

            - Sugarman

            - Younger/Goods

            - Perio Chisels(2)

            - Perio files (2)

            - Back Action

Pack of Curettes

      - ½ (no tape)

      - 11/12 (yellow tape)

      - 13/14 (Green tape)

      - 15/16 (Blue tape)

Extraction Kit

- Mirror

- Mirror

- Probe

- Probe/explorer

- Blade holder x3

- Goldman

- Molt

- Curved Molt

- Pritchard curette

- 4R/4L

- Prichard x 2

- Plugger/Plastic

- Packer

- Tissue Forceps x2

- Cotton pliers

- Hemostat

- Curved scissors

- Straight scissors

- Suction and hook

- Ligament knife

- Large and small elevators

- Root tip pick

Exam Pack

Mirror

Probe/Explorer

Sickle scaler

## Uncovering Kit

            - Mirror

            - Mirror

            - Probe/Explorer

            - Probe

            - Blade holder x 2

            - Goldman

            - Molt

            - Curved Molt (Molt curette)

            - Pritchard

            - Pritchard curette

            - Packer

            - Hemostat

            - Curved scissors

# How to Cement in a Temporary Crown or Temporary Bridge

- Have a tongue depressor and Temp-Bond along with the Temp-Bond mixing pad, plastic instrument, and cement spatula ready.

- Remove all cement from inside the temporary bridge.

- Check the fit of the temporary crown or temporary bridge in the mouth.

- Have the patient bite down.

- Use Shim-stock to be sure that natural teeth are in contact at the same time as the temporary bridge is in contact.

- Dry the teeth with 2x2 gauze and leave the 2x2 gauze on the teeth that are going to receive the temporary bridge.

- Dry the inside of the temporary crown or bridge with an air spray.

- Quickly mix Temp-Bond and place in the temporary bridge.

- Remove the gauze and place the temporary bridge or crown firmly on the teeth.

- Have the patient immediately bite down on a tongue depressor that is placed over the temporary crown or bridge.

- Immediately remove the tongue depressor and have the patient bite down and ask the patient if it feels okay.

- Return the tongue depressor to the mouth over the bridge and have the patient continue to bite down for about two minutes.

- Remove all cement using an explorer, curette, and floss to be sure that all cement is removed from the interproximal areas.

- Check the occlusion once again using Shim-stock.

# Impressions

## Taking Impressions using Kromopan material

- Set-up the room.

  - Alginator

  - Alginate

  - Spatula

  - Measuring cup

  - Blade

  - Tray-Plastic and/or metal trays

  - Tray adhesive

- Seat the patient.

- Place a large bib on the patient.

- Explain what is going to be done.

  - Ask if the patient has had impressions taken before, if there were any problems.

  - Explain the possibility of a gag reflex.

  - Tell the patient that if it happens, breathe through his/her nose and try not to swallow.

- Check the patient’s mouth

  - Check for tori.

  - Check what size tray to use.

- Try in the tray. The tray should not fit the teeth snugly. There should be sufficient room between the flange of the tray and the teeth to make sure that there is room for material. Therefore it should be slightly larger than the arch in order to be able to have sufficient room for material.

- On the tray that does fit, dry the tray with a paper towel. Spray the tray with Hold Alginate Adhesive.  On the tray that does not fit, put that into cold sterilization.  Allow the adhesive to dry for at least 3 minutes.  (When using Hold Alginate Adhesive make sure you are using two masks during the spraying process and you have a paper towel beneath the tray to block any adhesive from getting on the counter or cabinets as this could ruin it.)

- For a large  impression upper or lower, use 3 scoops of Kromopan.  For Small or Medium impressions, use 2 scoops of Kromopan. Measure out the number of scoops of Kromopan and tap the scoop with the spatula in order to make sure that you get rid of any air.

- Use the flat side of the spatula to level the alginate off with the top of the scoop, and put in a plastic cup.

- Measure the corresponding amount of water and put that in a plastic cup.  Note:  For 4 scoops you will be using a full measure of water plus another 1/3 measure of water and place that into a plastic cup.  Make sure the water is 73 degrees using the thermometer.

- Secure the bowl to the alginator.

- Pour the water into the bowl.

- Pour the alginate into the bowl.

- Using your spatula, wet all of the powder with the water. This should take only 10 seconds. You want to be able to make sure that the powder doesn’t fly out when you turn on the alginator.

- Turn on the alginator. Using the spatula, push the alginate against the side of the bowl. Alternate using the edge of the spatula and then the flat side of the spatula to get all of the material mixed.

- When the alginate starts to turn pink you are done mixing.  You have 45 seconds to mix.

- Load the tray.  Leave aside a little bit of alginate to have on your finger.

- Go to the mouth. With your finger, place the alginate underneath the lip

- When the alginate just turns from pink to white that is the time that you place the alginate with the tray in the mouth.

- For an upper impression, stand directly behind the pt., so that you are able to look down the nose and match the tray handle to the center of the nose.

- For a lower impression, stand in front of the patient and have the patient turn their head so that they are completely straight to you.

- Place the tray in the mouth, firmly place the tray over the teeth and seat the tray fully.  Note:  Your goal is to seat it fully without having the teeth touching the tray. This will require some practice.

- Grab the lip and pull it out over the impression tray

- Note:  For lower impressions, once you have placed the tray in the mouth have the patient stick his/her tongue to the roof of the mouth, tongue out and move the tongue from side to side for a few seconds to clear the alginate from under the tongue.

- Let the alginate become rubbery.

- Take the tray out of the mouth. The way you do that is to run your finger along the edge of the tray breaking the seal between the top of the tray and the tissue.  Then in one smooth and firm motion remove the tray from the mouth.

- Allow the patient to rinse and give the patient some towels in order to be able to clean his/her face off.

- Cut any excess alginate from the back of the tray.

- Rinse the impression and have the doctor check the impression.

- Spray the impression with multi-surface disinfectant spray.

- Wrap the impression in a wet, but not dripping paper towel and let sit for 10 minutes.

- Rinse the impression again and air dry.

- Before the patient leaves, check to see that all alginate is off the patient’s face and let the patient rinse his/her mouth.

- Type up notes in patient’s progress notes.

- Put the transaction in the computer.

- Fill out the Next Visit appointment sheet.

- Escort the patient to the front desk and have the receptionist schedule the next appointment.

- Doctor must check impression before the patient is discharged.

## Pouring Up Impressions

*If using “Silky Rock” stone, use 2 packs for 1 impression. Use the corresponding measuring cup that comes with the stone. For every pack used, fill the cup to the measuring line. (Ie: If using 2 packs of stone, you’ll fill water to the 2nd line on the measuring cup)*

1.	Add the plaster powder to the water in steady increments.

a.	Hold the mixing bowl in the palm of the left hand.

b.	Hold the spatula in the right hand. (or vice versa if left handed)

2.	Slowly incorporate the powder into the water with stirring motions for 20 seconds.  (Consistency of the mix should be like whipped cream.)

3.	Place the bowl on the vibrator on medium speed.

4.	Slightly press and rotate the bowl to permit any bubbles to rise to the surface.

5.	While holding the alginate impression in the left hand on the vibrator, use the right hand to place a small increment of plaster in the alginate impression, letting it flow slowly into the tooth indentations.

6.	Continue placing small increments of plaster/stone into the teeth while rotating the tray over the vibrator.

7.	After the teeth indentations are filled, use large increments to fill the palatal area.

8.	-If pouring up an upper impression, Use a paper towel border to wrap the impression

-If pouring up a lower impression that is NOT BEING USED FOR ANYTHING PERMANENT, make a tongue using ALGINATE or Wonderfill and pour up using a paper towel to border.

-If pouring up a lower impression that IS BEING USED FOR SOMETHING PERMANENT, fill the impression with stone and make a base with the stone on one of the tiles provided for impressions, flipping the impression on to the base.

9.	Empty the material into the impression until even with the paper towel.

10.	Clean the rubber bowl and the spatula. Remove as much stone as possible and place in garbage can, before rinsing bowl in the sink.

11.  The cast can be un-molded after 30 minutes, when no longer warm.

## Trimming models

1.	The cast is placed on the model trimmer and the bottom of the base is trimmed of excess plaster. Then the posterior is trimmed next.

2.	Turn the model to the side and trim it at an angle from canine to second molar on both sides.

3.	On the maxillary cast, trim the anterior from the midline to the canine at a 30 degree angle forming a “V.”

4.	On the mandibular cast, trim the anterior from canine to canine rounding it out.

5.	Trim the heel of the cast at an angle using the line on the trimmer.

6.	The casts are articulated to determine their symmetry and re-trimmed if angulation is not correct.

# Making Essix, Bleaching Trays, or In-House Nightguards

1.	Turn on Therma-Knife tray trimmer and set on number 6.

2.    Use the vacuum adaptation technique.

a.	Only use the vacuum in a well-ventilated area.

b.	Avoid electrical shock by not using unit near water.

c.	Check that the heater and vacuum switches are off.

d.	Open the vacuum adapter frame, position the sheet resin material in the center, and lock in place.

e.	Turn on the heater switch.  (When the heater core is cherry red, about 2-3 minutes, the unit is ready.)

f.	Raise the frame as far as it will go without forcing it.

g.	Swing the heater over the resin sheet material.

h.	Place the model on the perforated vacuum plate.

i.	Observe the sheet resin material carefully: when the material sags about 1/2”, lower the frame.  Excessive heating of the sheet resin material may result in fire or fumes.

j.	Turn on the vacuum switch.

k.	Turn away the heater.

l.     Loosen knobs on either side of frame and quickly lower material over model.

m.	Turn off the vacuum after about 15 seconds.

n.	After the material is sufficiently cooled, remove the model from the adaptation.

1.	Press the Therma knife around the gumline of the model (2mm below gumline for fluoride trays) melting through the plastic

2.	Remove the excess plastic from the model with scissors.

3.	Remove the appliance from the model.

4.	Trim the edges of the appliance with scissors and smooth with the blue polishing slow speed bur and/or sand paper.

When making an essix needing a tooth shade to replace a tooth:

*If tooth was present when the impression was taken, continue with the following steps. If there is no tooth on the model, wax in a denture tooth, form with stone, or if Itero scan has been taken, have 3D model printed. Then continue with the following steps:

            - Follow all of the above instructions

            - Take essix and use a slow speed with a 2 round bur to place a hole on the mesial occlusal surface and lingual surface.

            - Use chosen composite shade and fill the buccal surface with composite to appear as if a tooth was there. Light cure

If making essix for space maintainer and a graft or implant is being placed there:

- Take a little stone and place just over the area needing the space maintained. Keep in mind that if there is a healing abutment, it will stick up a bit, so the stone must be at least 3 mm high.

- Follow above instructions

# Hard Reline for Dentures

- The patient makes an appointment to see the doctor for a reline impression.  If we are doing a reline impression, the patient would usually like to have it back the same day.  Therefore, we should arrange the impression to be early in the morning so the patient can have the denture before we leave on that day.

- Doctor chooses the reline impression material.  Dr. Sheldon likes to use Coe-Soft.  Impression is made by the doctor.

- If Coe-Soft or Hydro-Cast is used (Hydro-Cast is something like Coe-Soft), the patient wears the denture with Coe-Soft/Hydro-Cast for about 24 hours.

- The patient then returns the next morning to see the doctor and asst.

- The assistant asks “Is the denture comfortable?” and then asks “Is there any pinching?”

  - If the patient says “yes” it is comfortable and “no” there is no pinching, the doctor confirms there is no rocking etc. Then the assistant puts the denture into a plastic bag for the laboratory.

- We then call the laboratory for a pick up.

- We should be able to deliver that denture by the end of the day that day.

# Construct Face Bows

1.	Place the face bow.

a.	Melt wax in hot water and place it on a bite fork.

b.	Have the patient bite into the bite fork.

c.	Fit the face bow into the patient’s ears.

d.	Tighten the knobs on the face bow.

e.	Fit the bite fork into the retaining rings (the small ring that can be tightened with 2 allen wrench screws).

f.	Place the nasion guide on the horizontal bar on the face bow and place it at the depression where the nose joins the skull and apply some pressure.

g.	Tighten the nasion guide.

h.	Use the allen wrench to tighten completely the two screws holding the bite fork.

2.	Remove the face bow.

a.	Loosen the top knobs only and remove the nasion holder.

b.	Do not touch the screws that were tightened by the allen wrench.

c.	Remove the entire bite fork and face bow assembly.

3.	Mount the face bow on an articulator.

a.	Place the ear holders into the small pins sticking out of the condyle of the articulator.

b.	Score the models.

c.	Soak the models in water for 15 minutes.

d.	Adjust the height of the bite fork so that it bisects the space between the upper and lower portions of the articulator. (It should be midway between the upper and lower.)

e.	Mount the lower model on the bite fork.

f.	Place retaining rings into the upper and lower portions of the articulator.

g.	Support the bite fork with a cup and/or newspaper so that the bite fork does not become depressed when the mounting plaster is placed.

4.   Finish

a.	Mix up mounting plaster.  (The mix should be thick enough that it does not fall out of the bowl when turned upside down.)

b.	Place plaster within both of the retaining rings.

- Pile plaster on top of the model.

# How to Make a Farley Appliance

- Take upper alginate impression of patient.

- Pour up impression per pour up instructions

- Put foil cote separator on model

- Using orthodontic acrylic, add powder around neck of crowns and add liquid and roll around on powder.  Continue to do this until it is a couple of millimeters thick.  Do for both the right and left sides.

- Then with a dappen dish add liquid, slowly incorporate the powder.  Add powder then tap on counter.  Continue to do this until acrylic is a doughy consistency.

- Then take acrylic ball and add to the anterior portion of the model.

- Allow acrylic to set.

- Then gently remove appliance from model using a buffalo knife.

- Once removed trim excess acrylic from the edges.

## Delivery to Patient

- Place in patient’s mouth and check for fit.  This should snap right in and stay in without assistance.  The patient should be able remove the appliance on his or her own.  Check to make sure it is not too tight.  If it is too tight, just grind away the points of acrylic that into the interproximal spaces first.  Then try-in. If that is insufficient, relieve the portion of the appliance that is against the teeth until the appliance seats.   If it is too loose, you will have to add acrylic.  If it rocks, you will probably have to remake it.

- After you check fit it is now time to check occlusion.  Have patient bite down and check with shimstock and articulating paper to see if patient’s lower anterior teeth are touching the bite plane.  You will need at least two, but preferably more on each side of the bite plane.

- Have Dr. Sheldon check.

- Polish on rag wheel with pumice and deliver to patient with a case.

# Nightguards

## How to Send a Nightguard Out

- Doctor is to take an alginate impression for the arch in which the nightguard will be fabricated; the assistant can take the opposing arch impression

- Bite registration may be taken with Blue Bite registration material by itself, or with a George Gauge bite registration. George Gauge can be used by using the bite fork and blue bite registration material on the fork.  If doing a Narval impression, George gauge needs to be set to 50-60% protrusion.

- You must have the pt. practice biting into the grooves of the bite fork before doing registration.

- Models are to be poured up and trimmed

- Enter lab Rx in Lab tracker and find out the lab this will be sent to. Doctor to sign the Rx

- Box up models and bite registration using bubble wrap and use shipping label etc.

- Call Fed Ex/ UPS for pick up

## How to Deliver a Nightguard

- Try appliance on model for fit.

- Try appliance in patient’s mouth for fit.  Make sure it does not rock.

- Use shimstock to check occlusion (bite).  Ask patient to bite down and check to see where patient has contact.  Then check using articulating paper.  On an upper night guard you want the buccal cusps of the lower teeth in occlusion with the appliance.  On a lower appliance, you want the lingual cusps of the upper teeth in contact with the appliance.

- Grind as necessary in order to make this happen or add acrylic if necessary.

- Check occlusion again using same procedure from line #3.

- Have the doctor check your work.

- Polish on the rag wheel with pumice and give patient a carrying case.

- GO over instructions with pt:

-rinse with tap water and use a wet toothbrush to scrub out the appliance

-keep out of heat

-keep out of reach of dogs

-If you find that your joints ache when taking out the appliance, chew a piece of gum for 5 minutes only to loosen up the joint, then apply a heat compress to the joint. Do this for 30 minutes, if the joints are still aching call the office.

- If you feel your bite has been altered and is not returning to the correct position after 30 minutes, call the office.

POT/OHI Regimen for all SB Cases (05/24/18)

(This can be found as a Quick Text titled “SB ”)

NV:------ POT 1 with MVF  1 week after surgery

1. Dental assistant irrigates under the denture using a monoject and has pt. demonstrate the action

2. Dr. Furtado does healing check and minor occlusal adjustment if necessary

3. Dr. Furtado will decide if POT 2 time needs to be adjusted.

NV:------ POT 2 with MVF 2 weeks after surgery.

1. Dental  assistant check to irrigate and teach the patient how to use Hydrofloss on the lowest setting only.

The assistant emphasizes that the Hydrofloss be used at the lowest setting only. 30 minutes (or whatever time you think the assistant needs.)

2. MVF--occlusion check and adjustment--2 units

NV:------ This appt. only necessary if it was felt at POT2 that pt. needs extra help with OHI. POT 3 no doctor 2 weeks after APPT 2 to check OH, increase setting strength of Hydrofloss. *at this appt, if no complaints, have patient continue to follow diet regimen in consent form and return at estimated time for Dr. Matt final impressions  (for ex: "Wait 2 months")

NV:------- MES discusses changes and does bite reg.

NV: Wax Try-in #1 (possible try-in #2) MES to do lightbody wash

NV: Possible wax try-in #2

NV: Deliver of Prototype

Adjust prototype, discuss any final changes w/ pt. before going to ceramic

NV: Deliver ceramic prosthesis and Deliver NG

NV: Adj. prosthesis as needed

NV: POT4

NV: PM

## Policy on post ops:

* At the end of each surgical procedure, ask the doctor if a post op is necessary and determine what the NV will be from there.

## Post Ops. (POT) and Set Up Checksheet for most sx. procedures (Ext, Imp. C/L, not Solid Bite procedures)

- _____   Have all clean barriers up including chair covers.

- ______ Have a bib with a bib chain hooked on the crease of the bib when unfolded and hooked to the back of the chair.

- _____   Have exam pack open , surgical scissors, cotton pliers ready  but not open and placed on blue paper.

- _____   Place several pieces of 2x2 gauze on the tray lined with paper.

- _____   Place a piece of dental floss on the tray lined with paper.

- _____   Place a dab of some toothpaste on a 2x2

- _____   Have a disposable prophy head attached to a contra angle and ready to go, if pt has teeth.

- _____    You may need to get out some OH supplies  for home care, Post OP toothbrush, container of floss and toothpaste. Proxabrush refill tips and holder. Place in a white lunch bag or a clear plastic take home bag, if pt has teeth.

- _____   Have a high speed, slow speed, saliva ejector, air water syringe loaded and wrapped and ready to go incase needed.

- _____   Have asst gloves and mask out on asst. side

- _____   Have Dr.’s gloves and mask out on Dr.’s side

- _____   Have progress notes up and read what the appt. is for and the last notes from the doctor.

- _____   Have Dexis up with the correct pt and the area up that was treated.

## First Post-Operative Visit (POT1) for Crown Lengthening, Implants, and Osseous procedures

1.	Have the room ready with an exam pack several gauze, floss, articulating paper, and cellophane (if applicable), prophy cup and paste ready and post-op toothbrush.

2.	Pull progress note up on Dentrix and Dexis.

3.	Place a bib on the patient.

Ask the pt. the following questions :

          - how did they do after the sx.

          - are they taking any ABX or pain med

          - is there anything that is uncomfortable

4.	Recline the chair and use a mouth mirror to observe the surgical area.

5.	Remove the perio pak, if applicable, and sutures (if EFDA, if not be ready for the doctor to remove sutures).

6.	Floss and polish(if EFDA, if not use a soft toothbrush to brush) the area with toothpaste where surgery was performed.

7.	Review plaque control instructions.

8.	Type in the patient’s chart.  (see VI)

a.	List any complaints.

b.	Write how the surgery area is healing

11. List any recommendations that the doctor tells the patient.

12. Fill out next appointment sheet.

13. If C/L was done, we must wait 3 mos. before going to a final tx.

14. See if pt. has paid for next phase

15. Ask Doctor what the NV will be and confirm that is what the blue highlighted sequence of NV are.

13.	Turn the patient over to the receptionist to schedule the next appointment.

## POT1 for STG and BG Procedures

- Follow regular post-operative procedures.

- Do not remove sutures around the graft and membrane unless they are loose and/or bothering the patient.

3.	Floss the area gently to the gum line and polish the area.

4.	Instruct the patient to begin using vista 2 weeks after the sx.  (no brushing or flossing the area.)

5.	Instruct the patient that he/she should not chew on the area for 6-8 weeks.

6.	Ask the doctor what the NV will be

# Pinhole Post Op Sequence

Need: mirror, probe, scaler, camera, retractors

Look at area, taking care not to pull down or up on lip

NV1: This is the day after the procedure and is with a photo-with no dr.

Directions to Patient: No looking, brushing, flossing, touching, swishing, spitting, drinking from a straw, no pressing, no ice packs, no periogard/vista

NV2: 1 week after procedure: POT1-polish using prophy cup or Q-tip depending on patient’s soreness, no floss, must see Dr., take photos

Directions to Patient: Pt. is allowed to brush only the lingual of teeth with soft brush. No looking, brushing, flossing, touching, swishing, spitting, drinking from a straw, no pressing, no ice packs, no periogard/vista.

NV3: 3 week check after surgery POT2-polish, floss, and photos (check with Dr. to see when to schedule the POT4)

Directions to Patient: Patient NOT allowed to brush/floss buccal, can only brush the lingual of the teeth. No looking, touching, swishing, spitting, drinking from a straw

NV4: 6 week check with Dr. POT3-polish, floss, and photos (check with Dr. to see when to schedule POT4)

Directions to Patient: Pt. allowed to brush and floss on their own

POT4

## Set up:

- Mirror, Probe, Floss, Articulating Paper, Cellophane, 2x2 Gauze

- Appropriate intra oral mirror

- Picture retractors

- Large Blue paper

- Camera (if using Canon, need to get the Casio or Dine Corps camera for the “Dr. and pt.” candid photo) and sign to take a candid with (if pt. approves)

- Ipad

- Appropriate XCP and Dexis

- Business card with blank side on back, Magazine, flashlight, pen

- T-Shirt (until these are used up), Cup, Success Story and Google Review paper and talent release

- Get “case photo” book

## Steps for a  POT 4:

- Gather info for doctor:

- Check to see who pt. was referred by

- Find out who did restoration (if any) or who GD is

- Ask pt. how the fit is, if they have any trouble eating or cleaning or any other problems

- Find out how long they’ve had the final work done

- See if they have any other tx. Planned  with us

- See  if they see us already for PM’s

- Take PA (if appropriate)

- Take photos and photo of pt. smiling in front of blue paper

- Give pt. success story and encourage them to fill out now.

- Put light on and get doctor

- Doctor will check with mirror, probe, articulating paper and cellophane

- Doctor will begin “graduation ceremony” (as dr. passes items to pt, take them from pt. and replace in the envelope they came in.

- If implant pt, doctor will do a candid photo with pt.

- When doctor leaves, you ,must get pt. to sign the talent release in order to use the photo for the digital photo frames, newsletter, website etc. Let pt. know what the photo will be used for and document in progress notes whether or not pt. agreed to photos and success story being used

## After pt. leaves:

- Download any photos

- Put name and info in Case photo book

- With implant patients, give ESS a note to send a letter about PM’s and Implants

- If the pt. agreed to a success story or photo, give a note to the assistant handling all photos.

# Re-evaluations (RE) Set Up Checksheet

- _____   Have all clean barriers up including chair covers.

- ______ Have a bib with a bib chain hooked on the crease of the bib when unfolded and hooked to the back of the chair.

- _____   Have an exam pack out opened with all instruments in order.

- _____   Place 4 pieces of 2x2 on left side of cassette.

- _____   Have packet of dental floss, articulating paper and cellophane paper out if needed.

- _____   Have a tooth slooth out but not opened if needed.

- _____   Have a high speed, slow speed, saliva ejector, air water syringe loaded and wrapped and ready to go incase needed.

- _____   If pt. is here for a RE of crown under a temp crown: Have temp bond, crown remover, cotton rolls, grit (sand), alcohol pad, tongue depressor, bur block, acrylic burs set to the side if needed for a re-evaluation for C/L if pt has a temp on.

- _____   Have asst gloves and mask out on asst. side of the table.

- _____   Have Dr.’s gloves and mask out on Dr.’s side of the table.

- _____   Have pen, pencil, notepad, tabs out on Dr.’s side.

- _____   Have Portable BP machine out if in a room where there is not one already.

- _____   Have an evaluation sheet out

- _____   Have progress notes up and read what the appt. is for and the last notes from the Dr.

- _____   Have Dexis up with the correct pt and the area that we will be re-evaluating. Make sure the x-ray is current. If it is not, then a current pa will be needed.

- _____   Have the CAT SCAN up and minimized if one was taken.  Have Invivo up with correct pt.

- _____   Have the box of Models in the room for the asst. to use if needed.

# Oral Hygiene Appointments (OHI ) and Set UP

- Have all clean barriers up including chair covers.

- Have a bib with a bib chain hooked on the crease of the bib when unfolded and hooked to the back of the chair.

- Have exam pack

- Place 4 pieces of 2x2 on left side of instruments.

- Place a dab of some toothpaste on a 2x2

- Have a disposable prophy head attached to a contra angle and ready to go.

- Have a plastic drinking cup half filled with water and a toothbrush laying on top of it.

- Get out some OH supplies out for home care, Post OP tooth brush, container of floss and toothpaste. Proxy refill tips and holder. Place in a white lunch bag or a clear plastic take home bag.

- Place 5 drops of disclosing solution in a cup with a splash of water in it.

- Have a tube of chapstick out.

- Have a high speed, slow speed, saliva ejector, air water syringe loaded and wrapped and ready to go in case needed.

- Have asst gloves and mask out on asst. side of the table.

- Have Dr.’s gloves and mask out on Dr.’s side of the table.

- Have progress notes up and read what the appt. is for.

- Have EAPT quick Text up and go through this with the pt.

- Have Dexis up

- Proxy brush

Policy and Procedure for Fixed in Prosthetic cleanings when removing prosthetics  UPDATED 07/17/17

Policy:

- Solid Bite patients will have cleaning regimens just like regular patients, from 3 month recalls to once a year, this will be established and decided by the doctor

- Permanent prosthetics will not need to be removed once a year. When the doctor asks for this, here will be the process: This process will include 15 min for MES or MVF to drill out (or unplug)and remove the prosthetic, the hygienist will clean the implants and MES or MVF will put back on the prosthetic and torque (another 15 min)

- Screws will need to be replaced every 4th cleaning.

Procedure:

Set up:

- Mirror, probe/explorer and sickle scaler

- C Sponge

- Cavitron

- High speed and 4 round bur

- Prosthetic kit with both star grip and 050 hex drivers

- Dappen dish, hydrogen peroxide and micro brush

- Teflon Tape

- TempIt LC

- Light

- Plastic instrument

- White stone

- Remove plugs in access holes using a sickle scaler, if it’s Temp It LC. If composite, use a 4 round bur to drill on high speed. Cavitron can also be used.

- Remove Teflon tape with explorer

- Use an .050 screw driver (short or long) and untorque. Then unscrew the screws. *NOTE: some screws may require a Nobel star grip driver*

- Place prosthesis in the ultrasonic to be cleaned.

- Get a dappen dish with hydrogen peroxide in it and use a microbrush to brush around the collar of the implant/multi-unit abutment.

- Remove prosthesis from ultrasonic cleaner and rinse, before screwing back into the mouth.

- Hand tighten all screws

- A doctor will step in and torque to 15NCM

- Place new teflon tape in access holes

- Plug holes with TempIt LC

- Use plastic instrument to make the material flush with the prosthesis

- Light cure with 3 beeps for every access hole

- If needed, a doctor will step in to polish with a white stone.

Fees for cleaning when prosthetics are removed:

$300 for the cleaning of a single arch- CHANGED 02/28/17 - it's now just the cost of a normal cleaning

$450 for both arches if done at the same time- CHANGED 02/28/17- it's now just the cost of a normal cleaning

$20 per screw replacement every 4th time – NO CHANGE- still charging for screws

# Quick Checks (QK CHK’S) Set Up Checksheet

- _____   Have all clean barriers up including chair covers.

- ______ Have a bib with a bib chain hooked on the crease of the bib when unfolded and hooked to the back of the chair.

- _____   Have exam pack out and a few 2x2

- _____   Place articulating/cellophane /floss pack nearby

- _____   Have a tooth slooth out but not opened if needed.

- _____   Have a high speed, slow speed, saliva ejector, air water syringe loaded and wrapped and ready to go incase needed.

- _____   Have asst. gloves and mask out on asst. side of the table.

- _____   Have Dr.’s gloves and mask out on Dr.’s side of the table.

- _____   Have pen, pencil, notepad, tabs out on Dr.’s side.

- _____   Have progress notes up and read what the appt. is and get anything else needed

- _____   Have Dexis up with the correct pt and the area that we will be re-evaluating. Make sure the x-ray is current. If it is not, then a current pa will be needed.

- _____   Have the CAT SCAN up and minimized if one was taken.

# Desensitizer Vanish Application

- Supplies needed are: cotton rolls, gauze, air syringe and Vanish or Gluma packet located in the lab cabinet over the dirty counter on middle shelf all the way to the right.

- Make sure tooth area is clean and flossed from food debris.

- Dry area with cotton roll or gauze. Do not spray air on tooth.

- Open packet of Vanish and tear off the top of material.

- Squirt onto the inside of packet.

- Use applicator found inside packet.

- Swirl around to mix.

- Apply to tooth structure at the root surface and into the margins facially and lingually.

- Apply a medium coat.

- Wait 30 seconds and apply second coat.

- Now you may use the air syringe to dry area if it does not bother pt.

- Make sure you use cotton rolls and gauze to pull cheek and tongue away from the area being treated.

- Pt should wait 1 hour before eating or drinking. patient should avoid brushing teeth for at least 4-6 hours after application.

- Pt may have to come back 1-2 times a week until sensitivity stops.

Torrens Powder

Torrens powder is excellent for Toning your gums.

Prepare as follows:

- 1 part salt (Celtic, Himalayan) use Epsom salt if you are sodium sensitive or are on a low sodium diet

- 6 part baking soda (preferably gluten free)

- Place in an electric blender and close lid

- DO NOT ADD WATER

- Put on a high speed for 5 minutes

- Leave lid on for another 5 minutes while powder settles

- Place in a sealable jar

How to apply Torrens Powder:

- Clean teeth

- Wet finger

- Pat powder on gums and let sit for 5-10 minutes

- Rinse with warm water

- Repeat daily

In the morning brush teeth, then apply Torrens Powder. Wet your finger and pat Torrens powder on gum line. If you use little enough, the bitterness is quite tolerable. Do NOT EAT, RINSE or DRINK for 1 hour. At bed time re apply the paste and leave on overnight. Use this paste for 6 weeks or until powder is all finished. When flossing, DO NOT go beneath the gum line, it will interfere with tissue which may be reattaching itself to the tooth.

# Emergency Appointments

- Ask the patient what the problem is:

  - How long has it been going on? Has this area ever bothered you before?

  - What area?

  - Is it getting better, worse or staying the same?

  - Is there a bad taste or metallic taste in their mouth?

  - What does it feel like? Is it sensitive to hot, cold or chewing?

  - Pain scale of 1-10, 10 being the worst, How bad does this hurt?

- Look at the area to see what is present.

  - Is the tooth broken, or is there a filling out, or is there a big cavity?

  - Redness

  - Swelling

  - Fistula

  - Pus

  - Ulcers

  - Mobility

  - Hygienist probe

- If needed, take an x-ray

- If sensitive to chewing:

  - use Tooth Slooth

  - check each cusp

- If sensitive to hot:

  - fistula?

  - chewing?

  - pulp test if no root canal

- If sensitive to cold:

  - Desensitize

  - Varnish

  - Use desensitizing toothpaste

7.    Take PA

- Call Dr.

- Give Dr. the information: Tooth #, show x-ray if taken, and results of your exam.

  - Diagnosis if you have one.

# Post Surgical Infection Protocol

-Are x-rays current?

-Have post-operative x-rays been taken of a surgical site?

-Are pulp tests necessary?

-What is the location of the infection?

-Is there suppuration?

-Is there swelling?

-Review medical history

-Diabetes?

-Last A1C and the result?

-Other medical compromises?

-Review medication list

-Osteoporosis medication?

-Steroids?

-Is the patient currently on antibiotics?

-Is there mechanical irritation from the prosthesis?

-What is the diagnosis?

-Were antibiotics given and finished after the surgery?

-What was the patient doing to care for the site?

-What has the patient been eating and drinking?

-Is the patient removing the denture, if applicable, sufficiently during the day and at night?

-Did the patient use Stella Life?

-Has the patient finished the Stella Life and need more?

Note: All infection patients should be seen within 24-48 hours for follow-up of the infection and determine whether it is getting better.

# New Patient Procedures

## New Patient Room Set Up:

Items Needed:

- 2x2 gauze

- Exam pack with mirror, probe/explorer, sickle scaler

- Maximum opening measurement triangle

- Dexis sensor and bar with ring and sensor holders for anterior,posterior, and BWX

- to the back: articulating,cellophane and floss pack and tooth slooth

- Photo retractors, fog free mirror system and camera

- blue paper

- bib and bib chain

- Consult sheet

- Gloves and mask for doctor and asst.

- Nomad if necessary

- Model box

-  MMPPI chart

- Have Dexis set up

- Go to patient tooth chart in dentrix.

- Click image at the bottom of the screen. Double Click. This will bring up a new dexis window- use dexis as normal.

- Go over medical history with patient

- Add med hx to chart by clicking the cross at the top of the chart. (It will be white)

- Click edit

- Add all medical alerts and allergies here. Once finished press “add to clinical note” this will add the alert to the top of a clinical note.

- Adding NP exam template –See picture below

- Find the note you just created- Go to the right side find “Clinical Exams” + then double click “Exam Type” this will prompt you to choose “initial”

- begin to fill out template

-When you reach the prompt to move to charting missing teeth follow these steps:

·         -Make sure your “light switch” is on and set to EX existing tx.

·         -Highlight the missing teeth all at one time.

·         -Find the Large RED button in your button list (towards the bottom) this will automatically place these teeth as missing in the chart.

·         IF that does not happen, check your lightswitch, check to make sure you are placing these teeth in existing. You can also highlight the teeth, press the big red M, and then go over to the existing button and press ‘EX’

Perio Charting

-Find the probe at the top of the patient chart. It will be a gray probe if the patient is new. It will have a blue box if the patient is not new.

-before you begin, make sure the chart shows the script set to “solid bite perio chart”

·         Go to Setup

·         Entry settings Setup

·         If you do not see solid bite perio chart listed- go to import from file

·         Find FSN1-server, file Tasha, Solid bite perio chart

·         Select open, then click “OK”

·         Select Solid bite perio chart in the script area in your perio chart

-Begin Perio charting as normal.

-The perio chart will first measure probing depths and then recession. You have to manually enter the mobility by clicking the tooth number and selecting a mobility

IF your perio panel is not found—go to view, perio panel

If your perio panel still doesn’t show up, go to your desktop and right click on any of the Dentrix application icons (ie: patient chart, office manager, appointment…), Select Open file Location, Open CUST folder, select Perio.startup.old, Rename this to Perio.startup.(whatever word you want), then refresh or close everything out and re-start Dentrix and this should be working now.

Follow the NP exam template

When the Dr is ready to treatment plan

-Make sure your light switch is set to TX for treatment planning

-Highlight tooth or area that the dr is tx planning and select the treatment from the Buttons OR procedure code list. This will automatically tx plan what button you have selected. Keep in mind that solid bite is a bursting code that will treatment plan every step needed for solid bite.

-If it does not automatically show in progress notes then check your lightswitch and tx button. You can highlight the tooth, select the treatment and then go over and press the “tx” at the top if you need to.

Once you are finished tx planning

-Add a NP exam- make sure your lightswitch is on “Complete” find and click the “NP” in the buttons box.

-Add FA code on buttons box as well

Printing your treatment

-Go to the treatment planner

-Phase your treatment by right clicking the treatment and placing a visit #1,2,3

-You may Rename you treatment by right clicking and clicking “rename tx plan” This is helpful when patients have multiple options i.e., “bridge option or Implant option”

-Now go to “Supporting information” and make sure the “financial policy” is listed in the case note template

-Press insert note if it is not inserted.

-You may type any financial agreements in this box and it will be printed on the tx plan i.e., “Patient agrees to pay half 3/2 and half 4/2”

-IF The financial policy is not there—Go to Settings and from the drop down menu of templates Select Financial policy. Go back and follow the steps before this to ensure it is in there.

--You are Now ready to print your treatment plan

-going back to the Treatment Plan Case set up, find the printer at the top of the screen

- Select “Print Treatment Case”

-Make sure that “Treatment Plan” Is under Select Report Options

- Once you have presented your treatment to the patient and they would like to schedule you may Accept and Reject the treatment cases by selecting the folder with blue arrow under treatment plan case set up.

-Change your Providers for each procedure by going to Progress notes, TX plan items, highlighting them and pressing the Dr with the Blue cross.

-If you patient decides to think about their options and come back later you will then go to the schedule. Double click on a blank area and start to create a new appt for the patient.

-Enter the provider, Go to TX and select NEW TX, Procedure-Select Adjunct Services, find your FA Code-highlight click ok, Press ADD then OK/POST,highlight and close. The treatment is now added to the appt. Add Staff member: that’s who will do the FA.  NOW press Wait/Will Call on the side. This will add the pt to the unscheduled list.  Select YES.

-If patient will not be returning. Go to patient file and change status to “Inactive” There is no reason to add them to the unscheduled list.

## Plaque Control

- Ask the patient what kind of toothbrush they use. Go over with them the difference between manual, mechanical, and the Sonicare.  If they are using the manual, show them that they should be using the manual toothbrush at an angle using small round circles getting the gums and gum line as well as the tooth structure itself and also show them how to brush all the chewing surfaces.  Just stress to them that Dr. Sheldon recommends the Sonicare and it is very effective for periodontal patients.  Have your patient demonstrate this.

- Flossing- Ask the patient what kind of floss they use.  Explain that we recommend using unwaxed floss and tell them about the following technique:  Tear off a piece of floss and wrap around middle fingers and use forefingers as a fulcrum and do an up and down motion getting below the gum line until you hear the squeak.  Once they hear the squeak they can move to the adjacent tooth surface.  Have the patient demonstrate this for you.

3.	Using a soft toothbrush and a model, explain and demonstrate to the patient how to brush.

a.	Brush toward the gum at a 45 degree angle using a circular motion.

b.	Go back and apply light pressure to get the bristles between the teeth using a small round motion.

c.	Do this on all buccal surfaces, then on all lingual surfaces.

d.	On the upper and lower anterior linguals, work the tip of the brush around the inner surfaces by holding the brush vertically.

e.	Brush the occlusal surfaces.

4.	Using a piece of unwaxed floss about 18” long, explain and demonstrate to the patient how to floss.

a.	Lightly wrap the floss around the middle finger on each hand.

b.	Hold the floss tightly between the forefingers no more than 1/2” apart.

c.	Gently insert the floss between the teeth using a sawing motion, being careful not to force or snap it into place.

d.	Guide the floss to the gum line curving it into a “C” shape against the tooth, moving the floss up and down on the sides of the tooth.

e.	Repeat this technique on each tooth until the floss becomes frayed.

f.	Turn one middle finger to the other to bring up a fresh section of floss.

## Implant Education

      - Go over tooth structures in perio booklet first.

      - Show patient picture of tooth and implant in jaw bone.  Now explain that the implant is simply a titanium post that is placed in the jawbone.  Explain the steps of the procedure.  Dr. Furtado will make a small incision and a drill a hole the same size as the implant and then thread the implant into place.  He will place a cover cap on. He will then suture the area up and allow this to heal in for 2-4 months to allow the bone and the implant to integrate.  After 2-4 months we will uncover the implant, meaning he will numb and remove the cover screw and place your healing abutment. This temporarily sits above the tissue and will allow for the tissue to form tightly around it.

      - If stitches are placed, we will wait 2 weeks and return for a digital scan for a custom abutment and final crown.  Explain the abutment is the part of the implant that sticks up above the gum line which the crown or bridge will sit on.

      - Show patient the diagram that represents the anatomy of the jaw and explain that sometimes grafting has to be done in order to place an implant.  Show them that they have a sinus and that we cannot place an implant in the sinus and that they have a nerve on the lower arch and that we cannot place the implant on that nerve.  If Dr. Furtado determines that you will need bone grafting, we will do the following.  On the upper he will just tease the sinus up a little bit and place some bone material there and let it heal in for 4-6 months and the same thing on the lower with the exception of the fact that there is no sinus on the lower.

      - A few weeks after the digital scan, the custom abutment and crown will be delivered.

## OSA(Obstructive Sleep Apnea) New Patient

- Have exam packet and models of sleep appliances.

- Go through MH the same as every pt.

- If pt. is referred by pulmonologist, be sure we have their sleep study

- See if they have a GD and see them regularly. If yes, ask how often and if they are in a current  treatment plan (fillings, crowns, etc). If they are, we must be sure all work is done before having appliance as it won’t fit after work is done.

- If they don’t have a GD, take FMX

- Take photo of malampatti score which is the back of throat or uvula

- (stick out your tongue and saw “AH”)

- Ask CMZ for the  “sleep apnea questionnaire” and Epworth Sleepiness Scale”

- Have pt. answer questions from these questionnaires

- Bring doctor in

- Remember, if pt. is a mouth breather, suggest the Narval appliance. If they are not a mouth breather, the Somnomed is a better fit. There is also a chance that the doctor will suggest the Tongue Bulb if the pt. wants to try it and are strapped financially.

# EAPT (Endoscopically-assisted Periodontal Therapy)

## EAPT Initial Visit

- The first Visit before a patient starts the procedure with the Hygienist is to supply the patient with all the tools and education necessary.

- The tools include the following or some of the following depending on what the Dr. wants the patient to have: Sonic Care tooth brush system, Water Pik, disclosing tablets, a one month supply of Juice Plus fruits and vegetables, a one month supply of COQ10 capsules, proxy brush holder and refills, dental floss and Vista. A carry bag to place all the supplies in for patient to bring back to every dental visit they are given.

- Each of these tools need to be demonstrated to the patient with the understanding that they can use them. Make sure before the patient leaves that they know how each tool and supplies that were given to them work.

- Encourage a healthy diet of fresh fruits and vegetables. Ask what their diet is like and give suggestions. Write down in the progress notes what they eat and drink for breakfast lunch and dinner along with snacks that they might have. Write down what they do for exercise or recreation.

- If the patient has not seen the EAPT video of Dr. Sheldon yet, show the video to the patient and ask if they have any questions regarding the video. (Show Dr. Sheldon’s EAPT video to all Perio Patients or any patient that you feel after the taking Xrays that will benefit from the video.)

- Go to Dexis computer and Click on the EAPT Video link on the desk top.

- After the video is shown, ask pt’s if they have any questions of what they just viewed.

- Do not let the patient leave with any unknowns. You want the patient to feel confident and encouraged about the treatment that they are starting and that this is a lifelong commitment and change for a healthier future.

## EAPT SUPPLIES

- Carry bag with a handle

- Sonicare toothbrush system

- WaterPik  system

- Conventional toothbrush

- Travel size toothpaste

- Travel size floss

- Interdental handle proxy holder

- One of each size of proxy head refills

- Super floss or Thompson floss if pt has a bridge

- Six Disclosing tabs

- One bottle of CoQ10, JP Trio- Fruit, Veg, Berries, Vitamin D

- Packet of information regarding EAPT instructions.

- Vista and explain the ratio

## EAPT OHI Post Visits

- At the first Visit, the room should be set up with the following:

- Liquid disclosing solution in a cup ready to go, chapstick, exam kit out with dental floss, toothbrush, proxy brushes, floss threader, several 2x2’s, toothpaste, prophy paste, prophy jet, water syringe, saliva suction, warm face towel for after visit.

- Greet the patient and ask them how they have been doing and if they are having any problems with the tools and supplies they have been given. Answer any questions they have. Explain to the patient what the visit today will consist of.

- Before the patient sits down have them place Chapstick on their lips so when they use the disclosing solution it will not discolor their lips.

- Have the patient swish with the disclosing solution for 30 seconds and spit in the sink. Then instruct them to rinse with clean water for 5 seconds.

- Seat the patient Pull the mirror down and have the patient see themselves in the mirror.

- Using a probe and mirror inspect the pt’s mouth and have them watch you in the mirror. Point out to the patient where they are missing. Ask the patient to demonstrate how they are brushing, flossing, proxy brushing, and Hydro flossing at home and how often. Show them the correct way and have them demonstrate back to you. Keep doing this until you feel that they got the hang of it.

- After the patient has been instructed, you should polish, floss, proxy, and prophy the patient Have the patient irrigate using bleach solution. Give the patient a warm towelette after you are done.

- Write in clinical notes using the Quick Text below everything and which visit they are at. include diet, what areas they need to work on. What supplies you gave them and so on.

The following is the Quick Text named “EAPT.” This should be double clicked and it will automatically put it in clinical notes:

1. Daily OH Regimen: How often is pt:

-Using sonicare?

-Using Waterpik?

-Flossing?

-Proxabrushing?

Problem areas found and recommended changes by assistant:

2. Diet over the last 24 hours (how much protein, fresh fruits and veggies? Look for high sugar intake, processed foods, fast foods, caffeine and soda)

Problem areas found and recommended changes by assistant

3. Is patient taking supplements and how are they taking them? How much do they have left? Do they need to place an order? (Co-Q10 twice a day, JP should be 2/2/2 of orchard, garden, and vineyard daily)

Problem areas found and recommended changes by assistant

- Tell the patient that they will come back each week for instruction until they have mastered it.

- Make next appt. or write down the next appt on the appt slip and give to the front desk. Encourage the patient as much as possible and hand them their next appt and dismiss patient

- The next appt will be all of the above.

# Oral DNA

- You cannot have the patient do an Oral DNA test if they have brushed, flossed, or used mouth rinse within the last hour.

- If patient is wearing lipstick, have them wipe it off with a tissue. Do not use a face wipe, this could contaminate the sample.

- If pt is wearing a denture, flipper or any other removable appliance, Have them take it out before they swish with the saline.

- Grab one box with the Oral DNA test tube and shipping supplies.

- Grab the brochure for the Oral DNA for the pt to take home.

- Open the box and lay everything out.

- Instructions are on the inside cover of the box.

- Remove the two labels inside the box and fill out with patients name and DOB.

- Using scissors, cut open the collection cup packet and remove the cap of the sterile saline. The saline must be clear, colorless and sealed.

- Put on your gloves.

- Hand the patient the saline vile and tell the pt to swish and gargle with the saline for 30 seconds then expectorate into the funnel of the collection cup.

- Remove the funnel and throw away in the trash.

- Tear off the top of the saliva preservative agent and squeeze into the saliva sample.

- Place the red cap tightly on the sample tube with the patient’s sample.

- Affix one label lengthwise on the collection tube. (Must be lengthwise for the lab to scan it).

- The other filled out label is to be attached to the chair side notes.

- Make a copy of the chair side notes, one to send in package and one to be scanned into the patient’s chart.

- Have the front desk schedule the patient back here for a  re-evaluation 3 weeks later or to be scheduled back for appropriate Root Planing

- Patient is not to start the scheduled Antibiotics until one hour before the Root Planing, unless otherwise stated by the doctor.

## Instructions for Fed EX Shipping Of Oral DNA

- Log into your Oral DNA account at oraldna.com  (login info can be found on FS1 under front desk folder and document called “logins”)

- Confirm an online test order has been created and submitted for each collected specimen. Select My Perio Path and Celsus One, let doctor tell you which one to select.

- Select all specimens that are ready to ship then create shipment.

- Print a Fed Ex shipping label and shipping Manifest, and schedule a Pickup.

- Place the box, folded shipping manifest, and test consent forms into a Fed Ex Clinical Pak. Seal pak, attach the shipping label, schedule a pickup.

VISTA

Vista (Bleach) Solution Protocol

Mixture: 1/2 tsp of Vista (Bleach) to 1/2 cup (4oz.) of water

Instructions: Swish (Soak for surgical) for 30 seconds, twice a week, no eating/drinking 10 minutes afterward. Do not store this mixture as it will lose it’s strength. Make fresh each usage.

This can be used in a water irrigator. If the taste is too strong to bear, you can place a couple drops of Listerine in the mixture.

(We will be using Vista which is slightly different than the household bleach)

Regarding EAPT patients:

Begin pt. on Valtrex 500 mg (one BID, for 10 days)

Oral DNA test

We will only introduce Vista solution the day we give the supplies. The instruction sheet is placed in the stock room next to the JP bottles, so be sure to include this little paper at that time.

At the EAPT OHI visits, we will do this last after having disclosed, done the brushing/flossing/proxa etc. Irrigate the mouth and problem sites with this solution.

Regarding Surgical patients:

When doing FA, we will be suggesting a 16oz. bottle of Vista (instructions are on the bottles).

(EXCEPT for tissue grafting patients, which will still receive the Closys rinse) at the POT1 or 2 weeks after sx. Unless otherwise stated by MVF.

Regarding PM patients:

We will be replacing the chlorhexidine with the Vista in a syringe only at the patient’s request. We will be providing the Vista here at the office for $18

Regarding Restorative patients:

Encourage this protocol when the patient is a perio patient as well.

*The solution will lose its potency after 2 days, so we will be making syringes and bottles every 2 days. This is up to the asst and hygienist to do when knowing they will be needing it.

*If the patient complains of the taste, you can suggest to put a couple drops of a mouthwash in the solution.

Stella Life

This is a homeopathic process that helps to lessen or even eliminate the need for pain medications.

Directions

Adults and children over 6 years of age. Start 3 days before the surgery or procedure to increase therapeutic levels. Continue for 7 days after the procedure. Apply 3-4 times per day, preferably after meals. Refrain from drinking or eating for at least 20 minutes after application. Use all three products at the same time in the order outlined. Consult your healthcare professional for use in children under 6 years of age.

STEP 1: VEGA Oral Rinse

■ Swish 7 - 10 ml (1/2 tablespoonfuls) for 1-2 minutes, expectorate.

STEP 2: VEGA Oral Spray

■ Spray 4 times under the tongue.

STEP 3: VEGA Oral Gel

■ Apply to the gum at the surgical area with a cotton swab.

Spray: 1 fl oz (30 ml), Gel: 1 fl oz (30 ml), Rinse: 8 fl oz (240 ml)

# A1C In-Office Testing

- The A1C in-office Test is kept in the refrigerator in Sterilization

- Get box out and take to Pt room.

- Grab the tester, Package 1, Package 2, and Lancet.

- Open both packages

- Place the cartridge from package 2 into the tester

- remove the white tip from the lancet by twisting and pulling it out.

- Prick side of finger.

- Hold clear piece from package 1 to the blood making sure you get enough to fill the entire middle tube.

- Place clear piece FULLY into black tube

- Shake black tube with clear piece in about 10x

- At this time, the tester should be flashing SAMPLE with a drop next to it.

- Remove plastic cap on the other end of the black tube

- Push all the way down on the white test spot on the cartridge and pull it away. Do not hold black tube on the cartridge.

- The display will count down from 5:00 minutes.

- After the 5min is up, you will have your A1C result.

# Financial Arrangements

## Checksheet for a smoother FA

Asst.

Initials

1.  _____    Have all the units added up for amt of time needed.  These should be written and highlighted on the FA sheet.

2.  _____    Make appointments and arrangements with other Dr’s if needed.

3.  _____   Look to see what dates and times work for the pt.  Write them down.

4.  _____   Make sure the front desk slip has been filled out by the front desk person for the visit and payment has been collected. Fill out the remainder of the slip with patient information for scheduling next visit.

5.  ____    Make copy of FA and consents that have been signed and gone over with patient

6.  _____   Put together surgery kit when needed.

7. _____   Have scheduling person make the appt and transactions.

8._____   Go over surgery basket with patient Give your card to pt to contact you for any questions that may come up.

9._____   Fill out the patient coordinator sheet/sheets for all that applies.

10._____   Have patient listen and watch JP video when you are out of the room for a few minutes.

11. _____ All financial arrangements must include the restorative doctor's name and a quoted fee.

12. _____ If a fee can’t be obtained by their dentist during the FA, you must provide the highest fee possible to patient, in writing.

13. _____ All “initial” areas must be signed by both the staff and the patient

14. _____ Patients must sign the FA and it must be scanned day of FA

15. _____ One FA person per patient. If you started the patient during their first phase as a NPP, they are yours forever!

Helpful hints:

1.  When bringing payment to the front desk for a transaction for an FA payment, give the front desk a copy of the FA with what is being paid and if there is any discount that was given so the front desk can properly add in the transaction.

# Check sheet for FA assistants

- Find patients wants, find out what patients fears are and budget (clear fears and give hope)

- Look at MH and make sure all is filled out, ask questions- obtain pharmacy

- BP

- Educate (consult pro, models)

- Pictures - print

- All needed xrays- print

- Ct if patient having any implants

- Add $185 for JP

- Write up  chart progress notes

- Put in transactions

- Make next appt or follow-up (a post op appt. is to be made, but not told to the pt. The surgical staff will ask the doctor in the room if a post op visit is necessary and cx the appt. if it is not.)

- Signed consents and FA

- Collect for surgery or do financing (add PM/Prophy if needed).  If patient does not accept treatment, it is under your own discretion to offer them a 5 or 10%  discount if they pay today or 2 weeks before surgery date if sched. You cannot offer this if they plan on doing financing with us.

- Give surgical basket

- Give pre-op for surgery *offer IV (and give pre-op)* Complete coordinator check sheet or sheets.

- If referring to another general dentist or orthodontist for a Solid Bite case or any other treatment, LNS will do all the treatment planning and will need a consult setup by the front desk for an online meeting to discuss the case.

- If patient needs a drug holiday or medical clearance, email LNS to remind him to dictate this letter, put this on coordination check sheet, tasks and alerts. Also, once SMW faxes this (after she dictates) she will place it in a folder to the right of the fax machine, front desk in charge person will attach and staple to the dictated letter and put back in your box for LNS to review and approve.

- When a tooth is extracted and an implant is placed, we need to make sure that there is a temporary partial denture or space maintainer that replaces the missing tooth or there is a nightguard that keeps the opposing arch in place. We are seeing too much evidence of super eruption which is actually changing the patient's bite. Please note this as part of your treatment plans.

- Patient Coordination shares duties with front desk (making pre-op calls 1-2 weeks prior to surgery, then front desk does confirmation call 1 day before surgery).

- There is a signup sheet above LNS/MVF desk for referring Drs.  Please fill out so the front desk can check the sheets each week and make meeting arrangements for LNS/MVF to discuss treatment cases of the patients.

## Updating Medical History at the time of FA’s

1.  	Make sure that the MH is not more than three months old for surgeries without IV sedation and no more than one month old with IV sedation.

2.  	Have patient fill out the Updated MH form and take their BP and Pulse and write them down on the sheet where it is listed.  If long version of MH is over a year old, a new one must be filled out by patient and signed and not the short version. LNS will have to sign off.

3.  	Make sure that you know and write down what medications they are taking and what they are taking them for. Write down all reactions they have for allergies. Be very detailed.

4.  	At the top of the form write down what ABX and pain meds that LNS/MVF wants them to take and the dose. Write under that if there is any lab work or any clearances that need to be done before any surgery can be done.  If patient is not paying but scheduling, find out when is good for them and let them know you need 2 weeks notice for them to receive the 5% - 10 % discount.

5.  	Place the updated MH form in LNS’S/ MVF’S box to be signed off. If he does not sign it off. It is to be kept and not scanned until approval is granted and shown to LNS/MVF to then be signed off.

6.  	Make notes in the chart notes and on the patient cord sheet.

7.  	When it is signed off give to front desk so they can add the data into the system and then placed to be scanned.

8.  	If a patient is diabetic and dextrose water is being used for IV drip then pt is not to take his or her Diabetic medicine the morning of the IV sed. Surgery or have non-dextrose IV water bag stated on the patient coordination sheet for the assistant to set it up for surgery.

## Guidelines and steps during and after an FA to make the front desk flow smoother

- After you have printed out the FA and have gone over all the paperwork with the Pt, you want to look at first available to sched. Add up all the units needed for Dr. and asst’s. If patient can only do certain days write them down and look to see what looks good for them. This is done in the FA room.

- If patient is paying, have money in hand and when you are ready to schedule, you will take FA, Money, Charitable donation money with sheet filled out to which organization and make a copy of the Fa for patient As copy machine is printing go get the surgery basket filled with Choc or Van at pt’s request. Grab copies of FA off the printer. Go to Scheduling person with all the info for when they want the sx, how many units and payment type. Write all this on the front of the FA TX sheet. This will help you and scheduler. Call in all the Rx’s if you have time when the patient is there or after they leave. Make notes where needed.

- If patient needs anything from a restoring GD, Ortho, Med release, etc, call and make them an appt for them before you start the scheduling process in step 2.  Sometimes you may not be able to make the appt for them.  If so Make notes, FU, Tasks and so forth when necessary. Look at Task Ck sheet.

- If patient is not paying but scheduling, find out when is good for them and let them know you need a 2 week’s notice for them to receive a discount. Let them know that you will FU with them and write down the due date on the TX plan that they get a copy for the patient to have a record.  Have the task, notes, and FU for payment in the computer before you leave the FA Room. Let the front desk know that you have done all this after the patient leaves. See task ck sheet.

- If a patient does not pay or schedule, ask them when you can call them back to FU. Place a FU or task (we strive for the furthest date out to be the end of the current week. Thursdays usually work out good for the asst.)

## If patient is scheduled and wants to pay at a later date – entering task

- We first strive for the end of the week, move to 2 week prior to surgery and last 1 week so we can order parts and materials.

- Put under “prepay tasks”. Date promised to pay and amount promised (For example, if it is a 2U ext. @ $610.00).

- Be sure to include any important pieces of information that will help when front desk needs to speak to them.

- Initials need to be on all notes that are added to the schedule and notes along with the date for that patient. Notes are in the left hand corner and when a note is placed and saved, it turns yellow. Be sure to delete a note and save when completed from the schedule.

Checking Codes and Dr. # For The End Of Each Day

1.       At the end of each day, the person in charge, will go to each FA that was done and look at the treatment plan to make sure that it is phased out correctly along with the Dr. that is doing the work.

2.       If it is not done correctly, a note will be placed in the FA assistants box to correct.

3.      Example: LNS, MES,  MVF,  Each treatment being done has to have a phase assigned to that Dr., this is how Eleanor knows what is getting paid to what Dr that is doing the work.

Sterilization

*Follow End of Week Maintenance Checksheet which contains Sterilization Maintenance as well

- Garbage is discarded.

- No sharps are to enter the sterilization room

- Ultrasonic

  - Instruments are brushed clean first.

  - Ultrasonic runs on three different cycles: light (5 min), medium (10 min), and heavy (30 min). This is determined by how big the load and what is in the machine.

  - De-gas setting is run every morning-per the instructions on the “opening the office” checksheet.

- Autoclave

  - Prepare the instruments for the autoclave.

    - Instruments are rinsed, scrubbed and dried, then wrapped with sterilization paper or bags.

    - Handpieces are cleaned using a soft denture brush (being careful not to get any water saturating them)and lubricated in the “oiling machine” and then placed in autoclave bags. ALL handpieces and motors are run in the Bravo autoclave only.

    - Cassettes that are wrapped with blue paper are as follows: Ext. kit, perio kit, uncovering kit, prophy kit, forcep kits and some restorative kits (there are certain restorative kits that are too big for small paper, yet the large paper is too excessive-these are the kits that get bagged. )

    - The sterilization steam tape turns to gray diagonal stripes when sterile. Ext. kits get both a small yellow and small orange piece of tape on one end of them. Upper Forceps are marked with an orange piece of tape, lower forceps marked with a yellow piece of tape. Prophy has their own tape and perio has their own tape. Everything else is marked with a sharpie.

    - Scissors and hemostats placed in cassettes in open position.

    - When cleaning the metal suction pieces with a brush, make sure you wipe off the tiles above the sink. Blood can splatter.

    - Gauze and C-sponges needing to be sterilized are placed in an autoclave bag and the “pouches” button is pressed on the autoclave sterilizers.

    - Implant kits and prosthetic kits are bagged.

    - BEFORE ANYTHING GOES INTO THE STERILIZER, BE SURE IT IS INITIALED BY THE ONE WHO WRAPPED IT.

    - Check water levels of autoclaves throughout the day and add more if the water level of midmark machines drops into the red

*Items that do NOT go in ultrasonic: cavitrons, castroviejo suture holder, syringes, any handpieces, osstell pegs

- Cold Sterilization

  - Place sharpening stones in cold sterilization

  - Place mouth rulers and most things plastic in cold sterilization.

  - Instruments are not reused for 12 hours.

- Changing and Updating Solutions

  - Cold sterile solution

    - Change every 28 days (Procide D).

    - Discard old cold sterile and rinse the container.

    - Mix new cold sterile and place in the containers in the sterilization room

## Maintenance Care of Lines and Bottles on Units

- Yearly Maintenance: Each operatory is equipped with a DentaPure stick inside each water bottle. This Stick is good for one year. Crosstex is the provider and will remind us to re-order and replace when the time comes. We will replace the bottles then at that time as well.

1

3

2

0

1

INCH

1

/4” - 1/2” From Bottom of Bottle

- Daily maintenance consists of being sure that the bottle is full of water from the back water cooler by the staff bathroom. Each morning, lines should be run for 2 minutes before seeing patients.

- In between patients, lines in rooms 1,4,5,6, should be run for 30 seconds before the next patient is seated. In rooms 2 and 3, every time after a handpiece is used, the lines should be flushed. To do this, you lift any two hoses and the “Flush Operations” selection appears on the screen. You select  “Between patients” and hang them into the sink. Every morning, the “daily” selection should be chosen and the same done.

- Weekly Maintenance: Using the Dentapure stick, there is no need for purging the lines. However, it was suggested by Crosstex that we clean out the bottle on a weekly basis. To do this, we’ll place a teaspoon of bleach in each bottle, fill the bottle up the rest of the way with tap water, swish the water back and forth, dump out the water, fill the bottle up with tap water, swish it around, dump it out and then refill the bottle with water from the water cooler in the back near the staff bathroom.

## Cleaning Statim

Once a week procedure

- Remove cassette from statim.

- Remove wire rack from cassette.

- Clean with sponge and a small amount of soft scrub.

- Rinse thoroughly.

- Dry off.

- Make sure rubber gaskets are free from cracks.

- Spray inside of cassette with statim plus spray. It is located next to the autoclave unit on the top shelf.

- Replace wire rack back in cassette and place back in unit.

- Check water level by removing cap on top left of unit.

- Fill with distilled water if necessary.  Use only Zephyrhills.

- Replace cap when done.

- Unit is now ready for operation.

## Cleaning Autoclaves

- Autoclaves are cleaned once a week

- Unplug autoclaves before cleaning.

- Remove all racks from units.

- Remove oval metal cylinder and solid metal bottom.

- Remove metal screen round filters located inside each of the units.

There are two each per unit. One is located on bottom back and the other located on the back of the unit.

- Place round filters in the small ultrasonic cleaner for 5 minutes to remove debris.

- Drain water from units by pulling the hose from the black hook located on the left side of the unit when the door is opened. Pour water into a bucket until unit is empty. Pour out water into sink next to units.

- Use a sponge with water and a small amount of soft scrub to clean inside and outside of the unit. Rinse with clean water and dry off.

- Be careful not to scrub on coil heater. The heater burner is located on the bottom of each unit.

- Clean all the metal racks in the sink in the same manner. Sponge, soft scrub. Etc.

- Place rinsed off ball filters back in the units.

- Place metal cylinder and solid metal bottom back in units.

- Place all metal racks back in the units.

- Hook hose back into the holder.

- Fill unit with fresh distilled water from Zephyrhills in one gallon jugs.

- Water level should always be filled to the top in the green area.

## Filling and Emptying Ultrasonic

- Large ultrasonic should be emptied at the end of each day.

- Filter under sink is unscrewed and cleaned out once a month from debris.

- Place filter back on unit after cleaning.

- One cup of ultrasonic concentrate is poured in unit and then clean water is filled up the rest of

| Purevac Maintenance Steps 1.) Aspirate 1 quart (32 oz) per chair of a PUREVAC® Evacuation System Cleaner Solution through the system at the end of each day a) Start with the operatory furthest from the vacuum pump and aspirate 1 quart (32 oz) of solution into the vacuum lines and leave vacuum open for this chair. b) Aspirate 1 quart (32 oz) of solution through the system at the next furthest operatory from the pump and turn off vacuum at that unit when complete. |  |  |
| --- | --- | --- |
| c) Continue procedure #2 until all operatories are complete. d) Go back to the initial and turn vacuum off. By keeping the vacuum on at the furthest chair it allows continuous flow of solution through the system to prevent any cleaner and/or debris from remaining in the lines. http://www.sultanhc.com/sw/swchannel/CustomerCenter/documents/6470/30623/Purevac_Maintenance_Video.mp4 |  |  |

Sterilization Lab Maintenance Checksheet

Sterilization Lab Maintenance Checksheet

Change Bio hazard bag under sink

___ Unplug ALL Machines and clean all autoclaves, Statim and Bravo machines.

___Drain all water (midmarks, Statim and Bravo)

___Replace with fresh distilled water

___Pull out little filter in back of machines and run in Small ultrasonic (Midmark and Bravo machines only)

___Scrub trays down w/ soft scrub (Midmark machines)

___Wipe down all machines in side with sponge and water only

___ Run Speed Clean through Midmarks

___Run test strips in ALL Machines

___Replace white towels on top of towel dispenser

___ Use stainless steel cleaner on ultrasonic machines

___Wipe down all counters/back splash

___Clean out and wipe down oiler machine

___Be sure all things are stocked and put away (any cleaning supplies needed to be ordered-goes to ESS)

Stock ice packs in sterilization frig

___Make any gauze packets, c-sponge packs, articulating paper packets, or sterile sleeves as needed

___End Of Day Friday: Drain large and small ultrasonics and wipe them out

___ Change batteries in door locks every 2 weeks

___Beginning of each month:

___Change cold sterile

___Hibiclens (pink solution in gallon jug) changed 1/2 solution, 1/2 water

___Milk bath solution changed

___Put floor mats from sterilization hallway

___Citric Acid

___In Upstairs lab: Take all instruments from their little drawer, run through ultrasonic and autoclave, making sure they ARE LABELED so they get put back in their place upstairs.

EOW Duties

___Handpieces and hot water baths from rooms removed and sterilized on Thursdays (Sterilize ALL in The Bravo)

___ Make sure Purevac is going through each suction line at end of day

___Help Shut down rooms (TV,computers,units) at end of day

___Check cameras at end of day for needed charging

___Laundry washed and put away

___Clean numbers on Door Keypads

___Calibrate CT scan and stock CT scan

___Pedals up on table, hoses wrapped up, chair raised

___All items off counter and put away

___Check expiration dates on all things

___Check all stock in cabinets/pull tickets for what is needed

Updated 09/19/19

EOW Duties for Each Room

Stock all Drawers:

____Air water syringes                             	____Saliva Ejector

____Small/Lg. white suction                   	____Big blue suction

____Proxabrush (allsizes)                         	____Floss threaders

____Thornton Floss                                    	____Super Floss

____ Regular Floss                                       	____Floss Picks

____Toothpaste                                           	____Post op Toothbrush

____Regular toothbrush                           	____Disclosing Tablets

____Bottle of disclosing solution           	____Horseshoe paper

____Cellophane                                           	____Thompson Sticks

Stock all cabinets:

____Topical                                                   	____Long and short needles

____Lido,Carbo, Septo, Marc                  	____Gauze packs

____Gauze/cotton rolls                             	____C-sponges

____Chlorhexidine                                      	____Monojects

____Cavi wipes____X ray covers____Handpieces to sterilization ____Barrier Sleeves ____Trash Bags ___Kleenex Box

____Check expiration dates on items when stocking. Place new stocked items on bottom

____Have x-ray unit head aiming upwards with new barriers (if applicable)

____Top off any opticide or disinfectant wipes

____Check glove boxes that they are full

____Wrap up BP cords

___ Clean pt. mirrors and lights above each chair

___ Check paper towels/soap dispensers/sharp bins

Stock the following:

____Consult/RE sheets                             	____MH

____Sx. Cheat sheet                                   	____Post op Sheet

____UMM                                                      	____Magazine

____Roll of tape                                           	____Scissors

____Roll of tape                                           	____Paper Clips

____Note Pads                                             	____Pencils

Bleach rinse done in each bottle:

____ ½ teaspoon of bleach to a full bottle of water. Swish back and forth, dump out, then fill bottle with tap water, swish back and forth, sump out and refill water from water cooler in back.

____Change trap in each room

Updated 06/18/19 *All above should be done by each assistant in their assigned room-LMW-rm1, LRA-rm2, KLD-rm3, ANP-rm6 (and sx. things in rms 2,3,6) and return this sheet to CMZ

# Change the Main Suction Traps

Make sure the suction is off in lab – red switch on wall

Follow these instructions to help in changing the trap: https://www.youtube.com/watch?v=hizMvcnSWwo

# Appliance Lab Upstairs

- Make sure lab is cleaned after each use.

- Wipe off counters and replace headrest bag over the stone vibrator unit. When finished with pour ups.

- When done using metal trays after model has been separated, remove as much alginate as possible. Place metal tray in jar that contains alginate softener. This will be emptied out every Thursday. Then trays are to be scrubbed and sterilized.

- Be sure all things are put in their place

- The trimming wheel should be clean of any stone

# Making Sterile Gauze

1.  Place 8 pieces of 4x4’s and 5 2x2’s in a sterile bag.
      2.  Place packet in autoclave
      3.  Set autoclave on pouches
      4.  Press start button.

# Making Sterile C Sponges

1. Place one  C- sponge in a sterile pack

2. Close and seal sterile pack.

3. Sterilized in the Autoclave or Statim.

4. After sterilization is completed. Stock in each of the rooms.

# Receiving Cases into the Office

Checking in a case:

Take the cases from the "cases in" box on the back shelf

Find the name on the physical case

Click the Lab Case Manager button:

Find patient’s name

Click the Receive Case button:

Click the “>…” and enter your initials to specify who checked in the case.

Check the case to make sure it matches the script in the “Case Note”.

Check the Case Status to make sure the correct status is selected.

Click OK.

Finishing a case:

After the assistant delivers the case to the patient;

Click the Lab Case Manager button:

Find patient’s name

Click the Finish Case button:

Click the “>…” and enter your initials to specify who delivered the case to the patient.

Check the Case Status to make sure the correct status is selected.

Click OK.

Attaching Case to an appointment from the Appointment Book:

While viewing the schedule, double click on the preferred patient.

Check the “Lab Case” box to indicate that there is a case associated with the selected appointment.

Attaching Case to an appointment from Lab Case Manager:

Open the Lab Case Manager button:

Click on the preferred patient

Click Edit then Attach Appt

Select the date and time that this specific lab case should be attached to.

Click OK to select the appropriate appointment.

Daily Case Check In

- Open case

- Look up NV for pt.

- Open Case Manager and confirm case + Rx match

- Check the box to the right of "Due Date"

- Type under the last line of RX stating the date and who checked in case (ie: CMZ 5/3/16 case received and placed in MES box to review)

- Find appt. on schedule (may be linked)

- Write on schedule "case here"

- (if associated with a sx. then be sure to complete in PC sheet as well)

- Write name and date of appt. on side of box

- Take out invoice and separate (if applicable). Place invoice back in box and pt. info goes on top or back in box.

- Put case in MES box to review

End of Day

- Pull cases for the next day and place on "Today's cases" shelf

- Make sure all lab cases are linked to the scheduled appt.

- All cases are here for the next day

- Models are all separated upstairs with notes to the asst. in charge that the case is ready

- Be sure all cases are being sent out and/ or boxed up

- Check the upstairs lab to be sure it is clean, straightened up and swept.

Weekly Case Check

- Make a list using the log in sheet (inside cabinet door)of all pt.'s that have a case needed for the upcoming week

- Write "here" on the log in sheet (inside cabinet door) if the schedule says "Case Here" AND you have double checked to confirm it is here and is what is says it is.

- Call labs for the missing cases and ask for the status on the missing case. BE sure to make notes about the phone call

Medication Protocol

Narcotics- For small surgical procedures give patient Norco 7.5/325 Rx for comfort med (unless patient is allergic to hydrocodone).  If patient is allergic to hydrocodone, give patient Meperedine 50mg AKA Demerol). In the case of multiple allergies consult with doctor on narcotic for patient.  For larger surgical procedures the patient will receive meperidine unless patient has an allergy. You must write this Rx out on a Rx pad and give to the patient. See medication sheet for reference.

Antibiotics-All patients having grafts done should receive a course of antibiotics.  Amoxicillin should be given unless patient is allergic. Clindamycin would be given if patient is allergic to amoxicillin.  If patient has multiple allergies consult with doctor.  Patients who are having extractions with no graft can receive 500 mg amoxicillin in an envelope or 300 mg Clindamycin in an envelope before leaving after consultation.  If patient is a premed (for heart valve reasons) patient, the following must occur:  2 grams of amoxicillin must be added to the amoxicillin RX.  For Clindamycin you will need to add the following if a patient is premed 300mg x 2.  If patient is not a heart valve premed patient, but is having a grafting procedure or implant procedure, give them the standard pre-printed RX for amoxicillin or Clindamycin.

Triazolam: The patient will normally receive three unless the patient is over the age of 60. This must be approved by LNS only.

*ALL MEDICATIONS THAT ARE NEEDED TO BE ON A HANDWRITTEN SCRIPT MUST BE CHECKED BY A DOCTOR IN E-FORSCE, THEN ENTERED INTO THE “PRESCRIPTION” PORTION OF THE PATIENT FILE *

If patient is a re-evaluation patient, you must update their medical history.

## Adding a Prescription

- Open the patient file

- On the Patient window, click the “Rx” tab.

- To add a prescription, click New and find the medication needed.

- Modify the Rx if needed per doctor recommendations.

- Be sure the correct provider is listed.

- Hit ok or Print

*ALL MEDICATIONS THAT ARE NEEDED TO BE ON A HANDWRITTEN SCRIPT MUST BE ENTERED INTO THE “PRESCRIPTION” PORTION OF THE PATIENT FILE AND A DOCTOR SHOULD CHECK E-FORSCE BEFORE PRESCRIBING *

# Taking X-rays with Dexis

- Make sure sensor is in place plugged into the HDMI port

- Be sure that Rinn and metal arm with appropriate sensor holders are hooked up correctly. Yellow-posterior, Blue-anterior, Red-BWX. Once the sensor is in the holder, cover using the handpiece plastic sleeve

3.    Go to patient tooth chart in dentrix.

4.    Click image at the bottom of screen. Double Click. This will bring up a new dexis window.

- Double click on Dexis- radiation symbol

If this a new patient do the following:

a.  Click on the new button at the bottom of Patient Administration Screen.  This will open patient data.

b.  Enter the patient number.

c.  Hit tab key.

d.  Enter patient last name.

  - Hit tab key.

  - Enter patient first name.

  - Hit tab key.

  - Enter birth date i.e. mm/dd/yy.  Do not use dashes or separators

  - Enter male or female

  - Click on done

  - Double click on patient’s name

  - Screen should be blank with patient’s name in upper left hand corner.

  - Make sure x-ray unit is on correct setting-should be about 20 (using a wall unit). Turn down if child or small framed person.

  - Be sure the upper left square is selected when clicking on the “cross hairs” symbol

  - Click on x-ray (radiation symbol)

  - Radiograph tooth chart will appear

  - Pick tooth or teeth that you plan to x-ray, click on that tooth. If you are taking an FMX, click on FMX and Dexis will give you the order. Click vertical bitewings for bitewings.

  - Tooth will then turn yellow once chosen (or BWXs etc)

  - The system then informs you to trigger the x-ray unit by turning the bars green.

  - Take x-ray

  - Image will appear on screen

  - Rotate if necessary (push R on keypad or rotate tool)

  - Check x-rays for brightness, enlarge by clicking on x-ray, and hit B to brighten.

  - To print an FMX you must do the following: click on the pencil icon, choose FMX with three anterior shots by using the down arrow, click select series, and then click the print button and choose one copy.

  - After taking x-rays bring the XCP's to the sterilization room and place in Sonic's. XCP's are then rinsed, bagged and placed in the Auto Clave for sterilization.

Additional guidelines:

    - The entire x-ray head must be covered with a dry cleaning bag.

    - The button that we touch should also be covered with a disposable adhesive plastic cover, the same as that which we use on the light handle.  By doing that, we are then touching plastic materials which then can be discarded.

    - If taking a single PA you must select all of the teeth numbers in that section. (see below illustration-ie: wanting to take PA of #3, you may either select #1,2,3 box, or #3,4,5 box. This keeps the X-rays streamlined and neat in the folder. Making it much easier to view.

NOMAD

When using the Nomad, you still need to use the Dexis sensor the way you would with a wall mounted unit. The sensor should be barriered, plugged into the USB port and the correct x-ray holder selected (Yellow-posterior, Blue-anterior, Red-BWX).

A lead vest is to be worn by the pt. and all others should be out of the room.

To Lock, so you don’t accidentally radiate:

Hold down the hand button, then the person button. When you are ready to use, you will unlock by doing the same thing.

- Press the tooth button to change from Anterior, Posterior, and BWX.

- Press up or down arrow to appropriate radiation time.

- Keep radiation time set to .21 seconds. If pt. is smaller, you may lower, if you get a red dot in the corner of your x-ray, raise the radiation using up arrow, If you get a blue dot, lower the radiation time using the down arrow.

*Note: You must hold down the “trigger” long enough or you will get a red dot*

- The Nomad has two batteries. These batteries are also the Nomad “handle.” Switch one out and put on charger when the other is low.

- Keep in mind the area of radiation as show below and stay within the “safe” zone.

Radiation Badge

- Radiation Badge is to be attached to the Nomad at all times unless using it.

- Assistant must clip the badge on her pocket or collar when using the Nomad.

- A monthly check of the badge is done by the assistant in charge by logging on to

www.instadose.com

user name is :    leensheldon  ( all lower case)

password is :      Gums321 ( the G is the only capital ).

Contact # for Instadose is:  877-477-5486

# ICAT

Settings for taking a CAT SCAN are the following and should not be changed for any reason unless the Doctors says to for a particular patient. If this happens, the settings need to be put back to the correct setting after taking the CAT SCAN, otherwise they are not to be changed.

1.       14cm. Diam - 8.5 Height

2.        3.4  Voxel -    8.9 Seconds

## Taking a CT scan

- ICAT VISION Q (right side)

- Place head rest and chin cup on arm.

- Chair must be placed at the highest level slot for patients that are under 5 feet tall. Feet must be resting flat on the foot stool.  Chair must be placed at the lowest level slot if patients are over 5 feet tall.  Note: If patients are extremely tall or are hunched over due to Osteoporosis, then the patient may have to slide down in the chair to be adjusted to the correct position.

- Seat patient and have them have their shoulders straighten back, place feet on foot stool.

- File

- New Pt

- Add

- Must place name, patient ID, Birth date

- OK

- Adjust patient to align correctly in machine as Pano format.

- Remember, butt and head should be all the way back. Loosen the knob on the arm so that the head rest is flush against patient’s head and brings head forward securing chin in the chin rest. Tighten knob.  The patient must be biting on their back teeth or closed down on their teeth for the correct bite.  Press preview button and align patient  using the up and down arrow buttons on machine.

- The vertical beam should be straight down the nose and the horizontal line should be across the mouth between lips. If the horizontal line is off, use the up and down arrows until this is achieved. Move the patient's head left to right to line up the vertical line.

- Have patient close eyes and be very still.

- Click OK

- Scan

- The horizontal dotted line on the computer monitor should be a guide to be sure that both upper and lower arches are parallel to this line. If the arches are not completely parallel, you will need to raise the chair using the up arrow and bring the head rest closer to the head.

- You will need to be sure that the “shadow” of the chin appears at least a ½ inch from the bottom of the preview screen. If it is not, you will need to raise the chin cup and the chair (using the up arrow).

- You will need to be sure that the “shadow of the nose” is showing. If it is not, you will need to bring the bar forward on the computer monitor to the left of the screen that says front and back.

- If you like what you see

- Hit Capture and ok and then press the scan button on the box outside of the room

Preparing the Lines for CT Scan and Transferring to Invivo

- Once CT has fully loaded, select both Maxilla and Mandible check boxes.

- This will now give you the Red and Green lines. To select an arch, left click on the appropriate line and move up or down a bit.

- Looking in the bottom, center panel, left click on the green line and move the line to the MCJ area. You will see the bottom right panel change as this is done.

- Move to the bottom right panel and begin placing the blue dots in the center of the arch. Making sure that that lines are following the contours of the ridge.

- The center blue dot should go between the centrals

- The blue dots to either side of the center one should go on the canines

- From there, continue to follow the contours of the ridge

- For the upper arch the lines should end by coming off the arch and tipping in towards the palatal vault

- For the lower arch, the lines should end by coming straight off the arch as if the arch continued, but not so far that it is off of the panel.

5.	Once complete and wanting to load to Invivo, go to the Highlighted CT under File Type to the far left and right click on the highlighted CT you are currently working on.

6.	Left click on Select Invivo Dental and it will ask you if you’d like to save the scan.

7. 	Select yes and name your CT scan. As soon as you do this, Invivo will automatically load

## Edit patient information on Icat

- Click on file upper left corner, click on new patient.

- Go to the patient you want to edit.  Click on the name of that patient and it should be highlighted blue.

- Click on edit on the right hand side (make your changes).

- Hit ok.

- Then exit out.  This is how to change or make corrections to a patient.

## CT Scan of Denture Only

Remove head rest. Place flat white square arm in slot. Follow instructions on tissue box. Place paper box covered with barrier. Place denture on top of box. Press alignment light and see if denture is centered. Take preview shot. If it is fine take cat scan as normal. If not make adjustments and take another preview shot.

## Taking a ½ CT Scan

There are some instances when an accurate pano can not be acquired, i.e. hunch back, wide shoulders, no neck, and overweight. This is where you would do a ½ CAT SCAN.

- Place head rest on and have it all the way pushed back.

- Place chin cup on arm.

- Follow steps in seating patient and aligning up patient in the Pano format.

- Volume

- Size of reconstructed volume- change to 8.5x.8.5

- Size of resolution change to .3 voxel-4.8 seconds.

- Hit preview

- Make necessary corrections as in the Pano format.

- Hit Capture

- Hit scan button on wall outside of CAT SCAN Room.

- DO NOT SAVE THESE SETTING WHEN DONE WITH SCAN.

## Taking a Pano

- Remove headrest from CT machine.

- Place Bite stick with barrier and chin rest on arm.

- Have patient remove all hair clips, eyeglasses, jewelry, nose rings, or metal removable mouth pieces if they have on person.

- Place lead apron over patient If patient is very hunched over do not use apron or it will interfere with pano.

- Click on ICAT vision ( rht)

- File & put pts info in or click on patient that already exists. If patient already exist right click on name and it should say require new scan.

- Have patient bite on stick and align on chin rest. Move the arrow up and down on arm to adjust position.

- Press preview button and ck red laser light for proper position. Light should run down the center of their nose vertically. Horizontal light should run through the corners of the mouth on both sides.

- Press the scan button

- If you like what you have press the capture button and then hit scan button again.

- If you do not like what you have during the preview, re-adjust and hit preview again and then scan until you get the correct position.

## Printing a Pano

- Open up iCat Vision

- Select the patient’s name

- Right click on big pano

- Select Open in Output folder

- Double click on one of the files

- Microsoft Office Picture Viewer will pop up and click print

- Another screen will pop up

- On the bottom uncheck Fit Picture to Frame

- Make sure that it goes to the cannon.

- Click print

- You will have to write patient’s name and date on the pano.

## Emailing Frames of CT using “Print Screen”

- Open ICAT Vision

- Select pt.

- Select CT scan

- Find exact frame or shot doctor wants

- On keyboard, hit “print screen”

- Minimize ICAT

- Open Microsoft office picture manager

- Right click and click “paste”

- Double click on image

- Go to right and click “edit pictures”

- Click “crop”

- Go to right corner of image and drag image in to desired frame

**If using a scan or frame for case photo, be sure pt.’s name is cropped out**

- Hit “ok”

- Go to “File”, “save as”

- Then re-name and select location

**NOTE: If e-mailing, open e-mail and attach file, do not copy and paste**

## Copying a CT Scan to a disc

- Blank disc’s are located on the top of the CAT SCAN machine.

- Write name with sharpie on the disc with pt’s name and date.

- Open disc slot on the side of the computer.

- Click on vision on the desk top.

- Bring up the pt’s last name on the left side. Click on the name top left.

- Click on tools, drop box down and click on export CD. Click on the square boxes on the left side that says CT and Raw CT.

- Click on create CD.

- It will take about 2-3 minutes for the CD to copy.

- A box will pop up that says CD complete. Click ok.

- The CD will pop open on the side for taking out. Place in a hard CD holder for mailing.

- Place copied CD in the pre addressed envelope on top of the CAT computer with the copy of the LNS signed form.

- CD envelope is weighed and mailed each Thursday.

- A copy of the weight chart is located in the second drawer in the large FA room with prices. Weighing is done with the scale located at Stephanie’s desk. Stephanie has a scale.

## RECONSTRUCTION OF A CAT SCAN

If you take a CAT SCAN and it is lost for some reason, you can reconstruct it from the Raw CAT SCAN image. This is so you do not have to take another scan on the person.  Here are the steps:

- Exit out of system

- Bring up the correct patient

- Right click on Raw CAT scan in the left Column.

- Click on Reconstruct on the left bottom when it pops up.

- Click box on Maxillary and Mandibular and then click on ok to contour the lines.

- Click save when exiting out of system.

## Calibration for the ICAT Machine

    - Turn on the ICAT Machine and the power switch on the wall.

    - Click on the Calibration icon on the Desktop.

    - Remove all objects from the Icat Machine. The headrest and the chin rest.

    - Click next on the screen.

    - Go to the power box on the wall

    - Click scan. Icat will let you know when to press scan again. This is done a total of 12 times.

    - When calibration is complete it will say on the screen, calibration is complete. Click ok.

    - Turn off Icat and the power switch on the wall to off.

    - This procedure needs to be done once every seven days even if the machine has not been used.

# Digital Camera

## Taking photos with Canon

- Make sure batteries in camera and flash are new and/or charged

- Turn flash on. Be sure Pilot light is on. If it is not, batteries should be changed out.

- Screen on flash should say “ETTL +1”

- Turn camera on

- Make sure large Dial next to on/off switch is turned to “AV”

- When taking a portrait of the whole face:

  - Stand back away from patient until you are able to see the whole face in view finder.

  - Using the dial (directly below the button to capture photo) adjust dial until “F11” appears on camera screen.

  - Then looking through view finder make sure face is centered and aligned using the dots/ squares.

  - If patient looks blurry, use focal adjuster on lens to adjust until the picture appears to be clear.

  - Press the silver capture button to capture photo.

  - Take a horizontal and vertical shot of patient

When taking a close up of mouth:

  - Using the dial (directly below the button to capture photo) adjust dial until “F32” appears on camera screen.

  - When taking intraoral photos, use photo retractors and mirrors when doing occlusal and buccal shots.

  - Then looking through view finder make sure area being photographed is centered and aligned using the dots/ squares.

  - If  picture looks blurry, use focal adjuster on lens to adjust until the picture appears to be clear.

  - Press the silver capture button to capture photo.

- Once photos are taken, you will need to download the photos, you can either use memory card or use the USB cable:

- First you need to bring patient up in Dexis, open their file and select the “+”                                                                                                                                                                                                                    at the top of the Dexis toolbar. Then select the upper right box.

  - MEMORY CARD: place memory card in card reader of hardrive of computer, wait for “DCIM” to appear on computer monitor.

  - Double click “DCIM,” select appropriate folder, wait for photos to upload from camera.

  - Once you see your photos, select photos (for more than one photo press “CTRL” on keyboard, then select the first photo, then go to last photo and select that one. This should highlight all the photos you need. For a single photo, press “Shift” on keyboard and use mouse to select photo)

  - Once photos are selected, left click with mouse and hold on any one of the selected photos. This will drag the photos. Drag these photos to the upper right box you selected on Dexis.

  - Lastly you will hit “delete” on keyboard to delete the photos from the memory card.

  - Be sure to remove card and place card back in the same camera

Taking photos using the Dine Corp cameras :

This camera has a dial next to the screen. C1 should be used for full facial photo and C2 used for intraoral photos. The settings are pre-set and should not be touched. If a photo turns out dark and you are on the correct setting, turn the dial to the A and then back to either C1 or C2 and try the shot again. You can zoom in or out to take your shot.

Camera comes with charger to plug in and is on the white cabinet in the hallways between the towel and blanket heater.

SD card is removed and placed in card reader as stated above.

Taking Photos using the Pentax Dine Corp camera:

This camera has already been pre-set.

When the power is turned on, it naturally zooms in. If you are needing to take a full facial photo, you will need to zoom in manually.

Taking Photos:

The following shots should be taken prior to tx.

- When using retractors, use large side of retractors for each photo except the buccal mirrored shots.

- Two retractors MUST be used for every shot needing retractors EXCEPT the buccal mirrored shot. One retractor is only necessary for that shot.

- When using the mirrors, get a cup of hot water and soak each mirror in this water for about 30 seconds, wipe off with a gauze, and be sure the mirror is connected to the handle well.

- If trying to get the shot of the upper arch: ask the pt. to hold the mirrors using the small ends up toward their eyes, ask the pt. to tip their chin down toward their chest and turn towards you, focus by pushing the shutter button half way down, once in focus, push the button down the rest of the way. (NOTE: if the camera has a hard time focusing while using a mirror, focus is on the pt.’s nose by pushing the shutter button down half way, then turn back toward your shot and press button down the rest of the way.)

- If trying to get the shot of the lower arch: ask pt. to hold retractors using the small ends and pulling lips down in the canine area, have pt. tip their chin up toward the ceiling and turn toward to you, focus by pushing the shutter button half way down, once in focus, push the button down the rest of the way. (NOTE: if the camera has a hard time focusing while using a mirror, focus is on the pt.’s nose by pushing the shutter button down half way, then turn back toward your shot and press button down the rest of the way.)

## Downloading Pictures to Dexis

- Take SD card out of camera and plug into SD slot on computer or card reader

- Locate folder and click “open folder to view files”

- Click DCIM

- Click 100…

- Press “ctrl” on keyboard and left click on the photos you want to save

- Once highlighted, hit delete on keyboard and then enter

- If wanting to erase all the photos consecutively, press”Shift” then select the first photo you want deleted and the last one in consecutive order.

- Hit delete on  keyboard and then enter

## Changing Flash Batteries

* This needs to be done as soon as you notice the ready/test light does not come on as

quick as it should!

- Open battery compartment on right side of camera.

- Remove batteries from the camera and place the charged batteries in (these are kept plugged into the wall in hallway).

- Place dead batteries in charger.

## Changing the Batteries in the Camera

- All batteries are changed every Thursday.

- Remove Flash rechargeable AA batteries located on the right side.

- Replace with 4 fresh rechargeable batteries plugged in the wall in the hall outside room 3 or in CT scan room. Recharge the ones removed in the charger for the following week.

- Remove AA batteries located on the left side of camera. These are non rechargeable batteries.

- Replace with 4 fresh AA batteries.

- Remove 2 123 lithium 3v. Batteries located on the bottom side of the camera.

- Replace with 2 fresh 3v batteries.

- All new non rechargeable batteries are located in a white drawer in the storage room marked batteries.

# SNAP

(1) Simply take a picture of your patient’s smile. Connect the camera to any USB cable, or use an SD card. Double click on the snap icon on desktop. If photo isn’t uploaded automatically, click the letter “C” then “A.” Create a new folder with patient’s name. Then import photo. Follow the blinking arrow, to walk you through step by step.

(2) Select the desired simulation, such as Whiten, Veneers, Close Diastema, etc. At this point SNAP will run its own automatic procedure. If more than one step is involved, SNAP will automatically select the correct tool for you, and prompt you with the next step. The typical amount of time you are using the software is less than one minute.

(3) Click the print button. Be sure this goes to the Canon Selphy. The automatic software will produce a convincing side-by-side comparison photo, complete with the name of your practice and telephone number (or any other information you would like on the print). Patients can take the handy pocket-sized before/after simulation photo home to show their spouse, a powerful advantage.

Surgical Guide Protocol for Fabrication of In House Guides

*Surgical guide fabrication for patients that have one or several implants, but NO edentulous cases for now.*

1st appt:

NPP exam including CT scan and FA (excluding Sx. Basket and Pre/Post OP instructions)

2nd Appt:

- Pre/Post op and Sx. basket given

- Digital Scan for surgical Guide (Be sure to select orthodontic or reference scan)

- Entered into Lab Case manager

*The above 3 steps should take no more than 6 units*

- Fill out sheet hanging over MVF desk

* MVF will design and print guide (this will take about 3 hours)

When guide has been completely printed:

- Using gloves, remove platform with guide(s) from printer

- Wipe resin off of platform

- Place guide(s) on soaking tray

- Immerse tray in first 91% alcohol for 10 minutes

- Immerse tray in second 91% alcohol for 10 minutes

- Place guide(s) in UV oven for 10 minutes

* We are giving 3 days to have the sx. guide complete

* FA asst. should place the PC sheet in MVF box to sign off on

Taking Videos

- If videoing a pt., have them sign a talent release

- Get handheld Dine Corp camera in top drawer of white cabinet in hallway

- Pull out audio pack and cables from a black mesh bag

- Plug large cable into connection on left of camera

- Be sure audio pack is switched to the middle setting

- Fasten lapel hook to mic

- Clip onto pt.’s lapel

- Turn camera on

- Use the menu on the Dine Corp cameras to  scroll down to the video mode.

- You can also use the Ipad to video

# Making a Video for Doctor

- Use hand held Canon video camera

- Hook it to the nice tripod

- Plug in charger to camera and into wall

- Be sure there’s an SD card in camera

- Power camera on

- Plug in microphone receiver to “AV/Out” plug

- Mic. Transmitter stays with the one being videoed and mic. Is clipped on lapel

- Be sure receiver and transmitter are on the same channel and power is on

- Press the red start /stop button on camcorder to begin and stop recording

## Upload Video to You Tube

- Insert SD card into computer (if using camera)

- Go to youtube.com

- Login: solidbitevideos@gmail.com

- Password: Thinkpink09

- Click Upload

- Browse files: Go to computer, SD card, find file wanting to upload and click open

- Re-name title to whatever LNS would like and list video as private, unlisted, or public as it pertains to video content

# Creating a Case Photo

**IF using a pt.’s face, you must have a signed consent (usually on POT 4) and call to confirm this is OK to do. Note in progress notes the verbal permission.**

- Open Microsoft powerpoint

- Delete the “click to add title” box

- Delete the “click to add subtitle” box

- Go up to “Design” and select a design

- Go to upper right and minimize powerpoint

- Open Dexis

- Find pt. and click “+”

- Click upper right box

- Click the export icon at top of program

- Next click “Desktop” and “JPEG.” These should light up orange when clicked.

- Select the square next to the number of the pic you want to export

- Click lower box that says “export images”

- Minimize Dexis and be sure photo is now on desktop

- Re-open powerpoint

- Go to top and click insert

- Click on “picture”

- To the left, click desktop

- Scroll down until you find the picture

- Click “insert”

- Picture should have immediately gone to powerpoint slide

- Move arrow to ball at UR corner of picture and click when double arrows appear.

- Drag left to scale to desired size

- Then click in center of picture and drag to desired spot on slide

- To add text

- Click “Insert” then click “text box”

- Click and hold on slide where you want text to go.

- Drag text box out

- Go to top tool bar and choose desired font and color

- Once complete, click”save as”, “powerpoint presentation”

- Select from left where you want file to be saved to

- Name file and click “save”

- Once done, open your e-mail and e-mail file as an attachment to Eleanor

Blue Note Communication System

This is another form of communication mainly used between the doctors and staff or those that choose not to have the walkie/headset system.

- When you are needing a doctor you simply left click on the doctor’s initials and room number. This lights up the slot alerting the doctor or front desk that they are wanted.

- You can click on menu and personalize these options above and also click on the setting allowing it to re-set by simply right clicking

- If you are wanting to send a message you simply right click on the slot you need and type what you need (ie: right click on MES room3 and type “pt. not numb”)

- Be sure you clear the light, once the action is complete. TO do this: either right click on the slot that is lit up, or left click and click clear light

# Closing Day Checksheet

Closing Day Checksheet

___Turn off all units in each room

___Re-start all computers and turn off monitor TV/screen

___Be sure Purevac has been run through each rooms suction

___All bottles are full for next day

___Turn CT machine off (wall unit) re-start computer

___Throw away all used barriers

___Put any models, dexis, camera, books, pens etc. away where they belong

___Place new barriers up and cover chair with new bag

___Empty all trash in operatories, sterilization, consult rooms

___Raise up chair, clean pt. mirror

___Sweep debris off floor and check for spots-mop if necessary

___Check laundry

___Be sure that all back doors are locked

___Wrap up BP/O2 cords

___Turn off O2 inside sterilization (flip lever)

___All machines running with dirty instruments

___All things put away from clean counter

___All items emptied from ultrasonics

___Large ultrasonic drained and wiped clean

___All items out of the oiler

___Light over sink off in sterilzation

___Wipe down the doorknob to back door and upstairs door using sani-wipe

___This checksheet in CMZ box

___Cameras on charger

___Blanket warmer unplugged

___Plug in wireless headphones to charge

___Drain stoppers placed over sink drain in rooms 4,5,6

___All handpieces taken out of all rooms

___All handpieces run through oiler and placed in the Bravo to be done by the morning.

Updated 10/2/19

# Ordering Supplies

- Remove the pink order card from the item to be ordered.

- Place pink card in clear plastic container marked “To be ordered”.

- Collect all cards to be ordered

- Categorize pink cards: Benco, BioHorizons, and general supplies

- Write these titles on top of each order sheet.

- Write your name and the date in the appropriate area.

- Fill out the form as indicated: manufacturer, product, company, catalog #, phone #, account, quantity, price, and total amount.

- Total the amount of each order sheet at the bottom.

- Fill out pink card of the day you ordered and the quantity and the price. This is a record of cost and when it has been ordered last.

- Call the company you want to order from. Companies are listed on the pink cards and on the front of the white notebooks.

- When the company answers, inform them you want to place an order.

- Give them your customer number so they can bring the account up on computer.

- Verify the doctor’s name and address.

- Find the charge card # and expiration date.

- Give the catalog number for each item and the quantity requested.

- Get the total dollar amount of the order.

- After the orders have been placed stick the pink cards in the appropriate white box slot in the stock until the shipment arrives.

- File the order sheet

- Keep a log of weekly charges.

- If something is needed that is not on the budget, a CSW sheet must be filled out and approved before it is ordered. This CSW must go to EL or the appropriate person for approval.

## Arrival of Orders

- When shipment comes in, check for perishable items first. Some items have to be kept in the refrigerator or freezer.

- When the order arrives, compare shipping quantity to the packing slip. Check each item with a pen and check mark.

- Pull pink card from white box and attach to one of the items for that pink card. Restock items in appropriate place.

- File packing slips in the hanging file marked for that company, if necessary. Most of these (especially the large companies) keep their own electronic copies that you may print off or view at any time.

# Opening the Office and Morning Lab Duties

Opening the Office and Morning Lab Duties

- ____Turn on O2 inside sterilization

- ____Turn on water and suction in sterilization

- ____(Whichever assistant has the first patient in that room, will open that room by doing the list below) Open Hygiene rooms if hygienists are not there that day

  - computers/TV in rooms

  - units in rooms

  - water baths in rooms

  - handpiece lines run for 2 mins

  - water bottles full (don’t forget the bottles in rooms 2 and 3 under the countertop)

- ____Turn on CT scan and computer

- ____Fill Ultrasonics

- ____Empty cold sterile, rinse all and put items away

- ____Make sure all autoclaves/statim have plenty of water

- ____Empty statim jug of all water. Then refill water up to the coils

- ____Make sure exam packs, cotton pliers, curettes, scissors are in each room (hygiene rooms if     they will be used this day)

- ____Turn on facial towelette machine and blanket warmer

- ____ Take garbage cans out to curb on Mon and Thurs.

- ____Wrap and run any dirty instruments/cassettes

- ____On Mon: fill baby wipe warmers with 4x4 and Hibiclens

- ____ Check stock in sterilization frig with ice packs

- ____ Place Wypall towels out on counters

Updated 8/12/19

# Compressor co2 tanks

- Co2 tanks are located in the compressor room outside the front parking area behind room 3 operatory. The key is located at the front desk.

- There are two green co2 tanks located in the room. There are valve turn handles on top of each tank. (Looks like a outdoor water faucet handle).

- Slowly turn the valve knob to the left on the Co2 tank that is hooked up to the regulator. Regulator is the item that has two Gages on them that shows the pressure of the tank and the amt of Co2 in the tank.  You should see the top gage dial move to around 80. The other gauge to the left should be between 500-1000 K.

- If the tank is empty the Regulator needs to be switched to the other tank.

## Switching the Regulator from an Empty tank to a full tank

- Turn the valve to the right to shut off the flow to the tank in case there is a little Co2 left in the tank.

- Take the wrench that is laying in front of the tank and loosen the nut that is tightened to the Co2 tank.

- Remove the Regulator and hand tighten it to the full tank as much as you can. Then take the wrench and tighten it up.

- Turn the valve to the left slowly until it’s all the way open.

- Top regulator gage should be at 80. Left regulator gage should be between 500-1000k.

- Call Rose Carbonic’s and tell Steve that you are on the second tank and need a delivery of Co2 ASAP- Steve is the contact and his office # is 632-7301. Cell # is 917-7253. Steve usually delivers on Tuesdays.

# Laundry

- Washing Machine

  - Wearing gloves, place laundry in the washing machine until it is half full.

  - Put ½ cup of soap (Tide Free and Sensitive) in washing machine.

  - Close the lid, press power, favorite, and start.

- Dryer

  - Clean out lint filter

  - Place clothes in dryer with a dryer sheet.

  - Shut door and set the temperature on regular high heat

  - Turn the far left knob to automatic more dry.

- Put away clean laundry.

  - When the dryer buzzes, remove the clothes.

  - Fold the blankets and hang the uniforms and coats

  - Fold the surgical cloths.

# Office Policy on How to Correctly Handle Sharps and Bio Materials

- All sharps are to be placed inside one of the pyrex dishes during a surgical procedure otherwise, should go directly into a red sharps bin. Syringes that still have needles attached stay in the same spot on the table for each surgery . Blades still on the blade holders as well.

- Recapping needles, this can be done by sliding the syringe needle into the cap, not touching the cap when doing so to avoid missing and being stuck.

- When cleaning up surgeries, sharps are the FIRST to go, remove all sharps before starting to clear the table.

- Sharps are to be removed in the operatories only, never in the lab.

- Heavy duty safety Nitrile gloves can be used at every clean up

- Dispose of all bone, blood, tissue, teeth, bloody tubing, bloody gloves, soaked bloody gauze or blood soiled items in the bio hazard red bags. Per OSHA regulations, these bags must be goose neck tied, not tied as if you were tying your shoe.

- Every surgery and hygiene set ups need small bio bags as part of the set up. These need to be within reach so you can conveniently discard of this material as you receive it.

- Blades are to be removed by hemostats, not by hand.

* You want to treat every surgery as a high risk situation. All staff must take the time to assure you are going by this policy during every surgery to protect you, the patients and other staff members. If you choose otherwise, it is in your responsibility to handle any accident.

# Drug Protocol

- At the beginning of each year, all drugs must be inventoried. The drugs in cabinet and emergency kit must have their expiration dates logged. Any expired drugs must be logged and placed in the correct Cyntox waste containers as listed below.  All others are logged on a sheet designed by assistant and each month they are gone through.

- CMZ has created a log for the following drugs: Propofol, Fentanyl, Versed, Demerol, Ketamine Hydrochloride, Odansetron, Promethazine, Robinul, Morphine Sulfate.

- Once the above drugs are received, the following must be logged on the sheet: lot number, expiration date, drug name

- Then, using a sharpie, label the vials numerically. When these vials are empty or expire they are given to CMZ and she will write date used or expired under the sheet.

  - Example below:

  - Drug Name: Propofol

  - Lot #: 123456

  - Expiration Date: May 1, 2015

| 1. 11/2/16 | 2. 1/2/17 | 3. 1/15/17 | 4. | 5. |
| --- | --- | --- | --- | --- |
| 6. | 7. | 8. | 9. | 10. |

*Managers and Doctors to be the only ones with access to locked cabinets

## Cyntox BLACK WASTE CONTAINERS FOR APPROVED PHARMACEUTICAL WASTE

- There are three different black HDDS containers’ for narcotics. Each black container is labeled with the items that can be placed in them.

- These containers are stored in the sterilization room.

- Only items with measureable amounts of liquid in vials have to be placed in one of the black containers. If the vial is not measureable or empty they can be placed in the trash.

- Any expired medications are to be placed in the correct black containers.

- Documentation of disposal is to be written on the correct form before placing in one of the black containers. This document is in the sterilization room on top of the containers.

- The lid must be kept shut after each use of the black containers.

- Once a black container is full the lid must be locked and a scheduled pick-up is to be made by calling: 1-877-927-8311. You will need to tell them that you need a replacement container. We are allowed 2 pickups per year for the fee that we have paid. It may take up to two weeks for a schedule pick up to be done. Please make sure you account for this. The sticker on the black container will need to be filled out completely before pick up, and a copy of the log sheet will need to be given to the driver. Log sheet is in the Surgery book.

- When we receive the new containers, make a new label for that container with the appropriate medications that can be placed in that container.

- Additional information and stickers are located in the biomedical Waste Plan Blue Folder on the shelf near the back employee door.

- If there is an emergency spill, please contact HAZMAT AT 1-800-424-9300. CUSTOMER # 21132

## LIST OF MEDICATIONS THAT ARE TO BE PLACED IN ONE OF THE HDDS BLACK CONTAINERS

- Dexamethasone

- Robinul

- Phenergan

- Zofran

- Benadryl liquid for injections

- Any ABX medications

- Any inhalers for asthma, COPD and so forth

## LIST OF MEDICATIONS THAT ARE TO BE PICKED UP BY AUTHORIZED DEA DISTRIBUTORS

RX REVERSE DISTRIBUTORS, INC 1-772-388-1212

This is the company that will need to be called for pick up of the following controlled substances:

- Fentanyl

- Demerol

- Ketamine Hydrochloride

- Midazolam

If these medications are in measurable amounts they will need to be placed in the container from Rx Reverse Distributors, Inc.

If they are empty or are not a measurable amount, they can be placed in the red biohazard containers.

## RED SHARPS CONTAINERS IN EACH OF THE PATIENT ROOMS

- Any sharps such as needles, blades, sutures, and IV catheters are to be placed in the red sharps containers.

- Carpules that are not empty need to be placed in the red sharps container. If carpules do not have any measurable amounts left in them they can be placed in the trash.

# OSHA Yearly Training

- OSHA yearly training is done each May in the early morning staff meeting.

- Folders for  each employee is given with all information with all materials needed to be covered. Each employee is to sign the inside of the packet with his or her signature and revision date that it was done. Topics to go over are the following: Blood borne Pathogens, Universal Precautions, Exposure Control Plan, Housekeeping-Cleaning and Decontamination, Sharps disposal, Regulated waste, Biomedical Waste, Laundry, Hepatitis B Vaccination, Labels, PPE’S, Post- Exposure Evaluation and Follow-UP, Procedures for Evaluation the Circumstances Surrounding and Exposure Incident. Recordkeeping for training Purposes, Sharps Injury Log. Use of the Fire Extinguisher. Xray badge that is to be worn with the Nomad.

- All of these are located up stairs in the lab  in a box labeled OSHA training.

- The Blood borne Pathogens Exposure Control Plan is on Laura Wideners documents file at home.

# Cyntox Waste Pick-Up

The Cyntox waste pick up book for record keeping is located above the microwave.  Cyntox will fax over the weight that is picked up on a three month cycle. This slip needs to be placed in the back of the folder for documentation. Once a month Cyntox will pick up the Biohazard waste and leave transportation pick up slip that also needs to be filed in the back of the book for documentation. To contact Cyntox call 1-800-424-9300.  They will also Pick up all medications that are disposed in a regular brown box. This will need to be called in for Pick Up.

# Material Safety Data Book

- Any new dental material that is bought and used into the office will come with a Material safety Data Sheet. (MSDS)

- This sheet needs to be logged and filed in the MSDS books that are located upstairs on the bookShelf in the lab that are labeled MSDS.

- These are used as a reference guide that has all the information on them  about that product for safety uses, disposal, what the material is made of, decontamination, disposal, whether the product is toxic or not. How to use and so forth.

# Using the walkie headsets

- Each staff member using the walkies should keep their headsets in their cubbie

- When entering the office each morning, grab a walkie and turn it on by pressing the power button. It will announce the channel when turning on.

- Plug your headset in to the side of the walkie

- The volume is controlled by the + and – sign on the side of the walkie.

- By pressing the button on the side of the head set, you can communicate to others in the office.

- If you are wanting to have a private conversation with another staff member, state their name only, the other staff member will then press the button on the very top of the walkie, the walkie will state to you and the other staff member Private conversation has begun. Once you are done with your conversation, you must not converse for 7 seconds and then the walkie will state that the private conversation has ended and return to a group setting.

- Rules to follow when using the walkie/headset:

-Say what you need to say. Say only the other person’s name if you’re wanting to have a private conversation with them (ie: “CMZ, can you bring a Thompson stick to room 3”) and get to the point. If it is too hard to communicate over the walkie, you can simply ask the staff member to meet you somewhere or have a private conversation.

-Use initials when calling someone, otherwise it is hard to distinguish between some names 	(ie: TLK and SAP instead of Tiffany and Stephanie, SJR and TCM instead of Sarah and Tara which sound alike through a walkie)

-If you are speaking/listening with a pt., do not interrupt them to answer someone on the walkie

-If someone does not respond to you, assume they are unable to speak back with you

-Do not touch your headset button with dirty gloves unless it is covered with barrier tape. (remember to remove the barrier tape between patients)

-Remain professional in your conversation so that patients can’t hear you and others don’t laugh  in front of their patients.

-When taking your headset/walkie off, state that it is off or you are out (ie: “CMZ out to lunch” /“ CMZ beginning a NP exam” )

-When returning the walkie to the docking station to charge, turn the power button off.

-Wipe down your actual ear piece at the end of each day before it goes back in your cubby.

# Maintenance Contacts

AC

Gary 427-6305

Pete 427-5772

Electrician

Mike Loftus 253-4333

Tom Tuttle (patient, did operatory lights, if you can't reach Mike) 428-5442

Plumber

Dave (general office stuff) 255-1577 office, 604-0347 cell

Hambel (operatory stuff) 242-9562

Handyman or could get other workers if needed

Marc 458-2050

Bug guy

Bill 298-6930

Appliance (washer/dryer)

Allied 254-4644 (leave message)

Roofer

Ed 704-2174

## Air pressured Hand Pieces

- When air pressured high speeds, slow speeds, prophy contra angles are broken they are to be picked up by George or his wife from Dentist Choice. Phone is 626 8563. Here are the steps.

- Give a note to Laura on what is wrong with the hand piece.

- Make sure that the hand piece has been sterilized and placed in the assistant in charge’s  inbox with the note attached.

- The assistant in charge  will then call Dentist’s Choice at the above number and if no one answers, a message will be left with the description of the hand piece and what is wrong with it. Message will also indicate that hand piece will be waiting at the front desk for pick up. Don’t for get to say what office you are calling from and leave your name and contact number.

## Electric High Speed Hand pieces

- When electric Hand pieces break or the electric Hand piece box breaks the steps are the following.

  - Call Greg Caudill. Home phone is 676-0620 or cell number is 446-6891. If no answer, leave a message for him to call back and describe what is the problem.

  - If Greg cannot fix it, then the hand piece and sometimes the whole unit box with the motor will have to be sent back to Brasseler. Brasseler’s Repairs department is 1-800-841-4522.

  - You will need to call them first and get instructions. They will usually send you a pre-paid returns label to attach to a shipping box that fits the item that needs to be shipped. You can find a box in the stock room or ask whoever is the office supply order or unpacking boxes to save you one.

## Operatories and Air compressors

If anything breaks such as the chairs, lights, table units, cavitron units, compressor, oxygen tanks, CO2 tanks, then you call Greg at home or on cell at the above number that is provided above.

## Oxygen and co2 tanks outside the building and portable oxygen tank in sterilization room

If tanks need to be refilled then call Rose Carbonics ASAP at 321-632-7301 or Steve’s cell number at 321-917-7253.

## Lawn Care Maintenance

Any kind of Lawn care that needs to be addressed and approved through a CSW and submitted to the office manager first with the CSW all filled out. Rich is the contact for the lawn service and his number is 321-759-2443.

## In-house Cleaning service

Any kind of cleaning that needs to be addressed to Cheryl by calling her at 698-2607 or emailing her at :  farfee4@gmail.com

# WALK THRU’S OF THE OFFICE

- Walk thru’s done at the end of each month and a list is written down on paper.

- Go into every room of the office and the building next door and write down things that need to be fixed, touched up or replaced. This includes the floors, walls, ceilings and upstairs.  This is an inspection process.

- Walk around building  # 1 and building # 2 and do the same thing. Include the sidewalks, parking lot, and the grounds.  You want to look at everything as if you are a visitor for the first time.

- Make a continuing list and email to the office manager and to Eleanor.

- Eleanor should respond to the email and say which items will be done and when.

- Keep the list on going until it has been checked off by Eleanor.

- When Eleanor has completed a task, she will write done next to it.

- This is the only time when you can take it off the list.

CHERYL'S MONTHLY CLEANING CHECK OFF SHEET

1. Cheryl has a monthly check list that is posted with the month at the back door area on the shelf above the microwave.
2. Cheryl is to check off weekly and monthly the areas on the sheet that have been completed.
3. At the end of each month, Cheryl is to place the check off sheet in Assistant  in charges box for review.
4. The Assistant  will go over sheet and if areas need more attention, The assistant will contact Cheryl by phone or by email.
5. Once areas and check sheet are completed then a new one will be posted for the next month.
6. This is a continuous list that can be modified.
7. The checklist is on the desk top of the large consult room for any changes that are needed.
8. Cheryl's phone # is 321-698-2607. Chery's email is:    farfee4@gmail.com

# Logging Google Reviews

- At the end of each month go to Google and look at our reviews through Google.  Click “Newest” when looking at reviews. You can also go to Yelp and do the same.

- Note the name and date of those who have given us a review that month.

- Then you can either go to GMR (getmorereviews.com) and see if the staff member “tagged” themselves when sending the invite, or you can go to Dentrix and look up the name of the patient and see the last staff member that saw them and then confirm with the staff member.

- Then you can go to the FS1 server and pull up the 2 documents to log/track. They are named Review Done and Reviews by Staff.

- Reviews Done is tracking each review,where it came from, the date it was done, and who it was by.

- Reviews by Staff is a monthly stat that shows which staff member has received a review. *WE DON’T TRACK HOW MANY WERE SENT, ALTHOUGH IT IS FOUND IN GMR *

- At the end of each month, we place the names of those who did a review in  a bowl and draw a name. The name drawn will receive a free cleaning. Be sure to call the patient and congratulate and place in the NV notes and on the face of the schedule.

GMR info:   user: drleesheldon123@gmail.com  pass: sheldon456.

TO see who sent what review: Log in, go to Invites, go to recipients, go to Inactive

# To send a Review from Desktop to Patient:

- Every computer should have the shortcut to GetMoreReviews on the desktop. If not,  open google browser and type in

https://reviews.localbusinessally.com/leensheldondmdpa/invite?invite_token=vm-Jgrp4esLVFesbsLBHig

NUCALM

- Place the calming cream on either side of patient’s neck (along carotid artery)

- Wipe behind the patient’s ear lobe using alcohol wipe.

- Place electrode pads behind the ear lobes

- Hook up the electrodes to the Electrostem unit (if this does not turn on, it may need another battery. The silver 9v batteries are rechargeable and kept in the CT room)

-Turn this on using the button on the side, press start, and keep the setting at 0.1 and the time can be modified to however long you want it to run through the procedure.

5. Hook up the headphones to the tablet

6. Turn on the tablet and press the Nucalm app

- Password is “relaxed”

- Select Start a New Session

- Press the play button on as many tracks as you would like to play (each are about 50 minutes in length).

7. Place eye mask over patient’s eyes

Creating a New Patient Appointment and Family File

First you will double click on the appointment time you are wanting to schedule the patient for. This screen will come up. Select “New Patient” in the bottom left corner.

The Next Box will show up: This is the patient information box Please fill in all of the boxes. If you are in a hurry and need to schedule quickly you may edit this information later.

Do not forget to include the Referred by-

Once you click OK it will direct you to the appointment block in order to schedule the appointment.

Beginning at the top of the box:

1.       Fill in provider for the appointment

2.       Click Initial and then NP EXAM

3.       Click OK

4.       Go to schedule and create an FA block for after the NP exam

5.       Right click the scheduled time and create event. Make it teal in color and for an hour long.

6.       Put Patients name, FA, with assistant’s initials on the event

You have now created an appointment for a new patient but you are not finished with this patient until the family file has been created.

Creating a Family File for a Patient with an Appointment

Click the appointment you just created and then go up to the family file

Click File- New Patient with Appt

# CHECKING A PATIENT'S BALANCE- FINDING PAYMENT INFO

- Go to the patient’s ledger

- You can check the patient’s balance by looking in the balance column. “–$$“ amounts are how much the patient has paid towards his treatment. Everything in blue is completed treatment.

- If you double click the payment “green” You can see the payment notes listed there. It will show up as a music note in the ledger payment

- When making a new payment note. Refer to the treatment plan to see which phase or treatment option that has been paid for.

- To find what needs to be paid—Click on the  Patient Alert button in the tool bar. There you will see an alert for the amount due listed under “FA” click edit. Now you can view the alert.

# Confirming A Patient

1. 	Select a Patient and double click

2. 	Click on “Status” and “FIRM”

# Create a NEW PC Sheet

1.        	Open PATIENT CHECKLIST on FS1 server and copy document

2.        	Find Pt.in dentrix

3.        	Go to Ledger

4.        	Select Options

5.        	Select Treatment Planner

6.        	Select Supporting Information (lower left)

7.        	Go to Informed Consent: Select drop down box and select “PC checklist” and click add

8.        	Click Additional Notes

9.        	Right click and paste document

10.      	(you can fill this out here, hit ok and save…OR….you can hit ok, save, and then open it in the doc center to edit. It’s up to you) ** NOTE: When you hit save, it doesn’t do anything *

11.      	Close

Editing an Existing PC sheet

1.        	Open a patient in Dentrix

2.        	Open the Document Center

3.        	Double click on patient name

4.        	Double click on Treatment Planner Consent Forms

5.        	Choose the last treatment plan listed

6.        	Click on the Modify Document icon at the top of the toolbar

7.        	Click “Additional Notes”

8.        	Fill in the blanks (you can just delete the blanks and type initials and date in. Also, to make it look “pretty” you may have to tab over or whatever to make it look more uniform)

9.        	Hit ok

# Printing a PC Sheet

1.        	Open patient in Dentrix

2.        	Go to Document Center

3.        	Double Click on pt. name

4.        	Double Click on Treatment Planner Consent Forms

5.        	Select by SINGLE clicking (or highlighting) the Treatment Plan you’d like to print

*NOTE: the most current plan is the one at the bottom-The date won’t change-that’s the date I set up the PC sheet, but the actual PC sheet will say the date of the last edit

6.        	Right click on the selected plan and click print

# Document Transfer from DSN to Dentrix

1.       Search desired patient in DSN and open attachments. Select “Documents”

2.       Select the desired document to be transferred. Click “View/edit” button at bottom of attachment window

3.       Select “File”, “Print”, select “Dentrix Document Center” as printer

4.       Repeat steps 3-4 until desired documents have been sent to the Dentrix Document Center

5.       Return to Dentrix schedule and select patient, go up to the toolbar and click the file cabinet that is the “Document Center”

6.       A window will pop up and ask if you would like to open the Unfiled Documents. Click “Yes”.

7.       A second window will ask if you would like to set up Automatic File Acquisition. Click “No”.

8.       In the toolbar, click the yellow file folder with an orange triangle that says “Unfiled Documents”

9.       Scroll to the date you sent documents to the server, and click the paperclip with the green arrow to attach to the patients file once you have selected the file.

10.   Change date to the date on the document and select the appropriate category and description of attachment

11.   Click “Ok” when finished.

12.   A window will pop up and ask if you want to “Change Document for All Attachments”. Click “Ok”

# Entering Patient’s Current Medication List

From family file or clinical chart:

1. 	 Select pt

2. 	Click on Medical Alert

3. 	Click Edit

4. 	Click Other from the list on the left

5. 	Click Note section to right

6. 	Click Insert Date Button (on same row as the Note: is located at top)

7. 	Enter medications -  type in each medication.  You will not find them on the left (These are just alerts)

# How to FindThe Patients Next Appointment

1.  	Click on icon at the top tool bar on the schedule that says “Locate Existing Appointment” (looks like magnify glass)

2.  	Type in patients full name

3.  	Click arrow to the right of the name

4.  	The appointments will appear

Or

1.  	Click on Patient or Type in name or find patient in patient file

2.  	Click the blue “i” Icon at the top tool bar

3.  	All appointments will display

# How to Create/Print Deleted Transactions Report

- Selection Office Manager

- Select Maintenance

- Select Audit Report

- Change Date under “Select Audit Date” (Upper Left)

- Select Staff member or members (Upper Right)

- Select Completed Procedures under “Select Changes for Standard Report”(Lower Left)

- Select ok

- Batch Processor will be the screen that appears after you select ok

- Double Click on Audit Trail Report

- Report will open up – Select printer(Upper Left Corner)

# How to follow up patients on the unscheduled list

1. 	Go to the Dentrix Appointment Book

2. 	Click on the “Appt List” on the top tool bar

3. 	Click where it says, “select list” and choose, “unscheduled”

4. 	The list is all the patients that are unscheduled yet due for an appointment

5. 	Go through the list and contact those due. See if they are ready to schedule.

6. 	Write up what you did in the chart notes

7. 	Either take the patient off the unscheduled list and schedule them or inactivate them OR keep them on the list and give good reasons as to why in the notes. *don’t hang on to the patients unless you have promising reasons to do so, they can always call us back when they are ready.

8. 	Be sure to take them out of the “unscheduled list” if you in activate or schedule.  Be sure to write notes in chart notes

The goal is to keep this list VERY small.

# Go over medical history with patient

-Add med hx to chart by clicking the cross at the top of the chart. (It will be white)

-Click edit

- Add all medical alerts and allergies here. Once finished press “add to clinical note” this will add the alert to the top of a clinical note.

-When you bring your patient back you will need to chart missing teeth prior to probing

Follow these steps:

•      	-Make sure your “light switch” is on and set to EX existing tx.

•      	-Highlight the missing teeth all at one time.

•      	-Find the Large RED button in your button list (towards the bottom) this will automatically place these teeth as missing in the chart.

•      	IF that does not happen, check your light switch, check to make sure you are placing these teeth in existing. You can also highlight the teeth, press the big red, and then go over to the existing button and press ‘EX’

# Perio Charting

-Find the probe at the top of the patient chart. It will be a gray probe if the patient is new. It will have a blue box if the patient is not new.

-Before you begin, make sure the chart shows the script set to “solid bite perio chart”

If it is not set follow these steps:

•      	Go to Setup

•      	Entry settings Setup

•      	If you do not see solid bite perio chart listed- go to import from file

•      	Find FSN1-server, file Tasha, “Dentrix” and click Solid bite perio chart

•      	Select open, then click “OK”

•      	Select Solid bite perio chart in the script area in your perio chart

-Begin Perio charting as normal.

-The perio chart will first measure probing depths and then recession. You have to manually enter the mobility by clicking the tooth number and selecting a mobility number in the perio panel

If the perio panel does not appear (with all of the numbers), go to View, then perio panel

# Entering your Hygiene Quick Text

-Select patient chart

-Go to the quick text bottom right of the screen and select Hygiene, select the type of visit

-Follow the prompts and begin filling out your template

# Adding a Transaction

-Change your light switch to “Completed tx” That’s the blue check mark

-Now Find the buttons that correspond with the treatment you just did for that patient

-It will most likely be found in these two columns of hygiene tx.

-Whatever button you click will now be added to completed work, check this by going to progress notes at the bottom of the tooth chart

# Making their next visit

-Double click their current apt on the schedule

-Click Schd. Next- a new apt box will appear

-Pick provider, Treatment the pt is getting and then pinboard the apt.

-Go to date you would like to schedule the apt and drag the apt from pinboard into time slot.

# Inputting and Updating Patients MH Information

1.  From the patients Family File, first check that the patient’s phone numbers, and address are correct to what is on the Medical History.

2.  Click the white or red cross that is on the patients Family File.

3.  Input patients list of Medical Alerts on the left side and any other Alerts that cannot be found on the left side and list the medications the patient is currently taking on the right side under notes.

4.  If the patient has Med Alerts that are imperative to know at each appointment, create a pop-up alert to show up on appointments and when the patients file is clicked on. These important Med Alerts are as follows:

·         Cancer/currently on chemo/Radiation

·         Anaphylactic Allergies

·         Allergy to Antibiotics

·         Allergy to Pain Meds

·         Dementia/Alzheimer’s/forgets things

·         Diabetic

·         Epilepsy

·         Latex Allergy

*Refer to the Dentrix HAT to create a Pop-up Alert

# Enter Payments and Place Transactions

1. Go to the Ledger (3rd icon on top tool bar)

2. Select patient

3. Go to the icon at the top tool bar with the tooth and left arrow for procedures

4. Go to the icon at the top tool bar with the money and left arrow for transactions/payments

OR

- Go to the ledger

- Go to Transactions

- Enter payment or Enter Procedure

- Select what type of payment was made- cash, credit card and type, check

- If Check, Place check number- Click ok to enter

- Entering a procedure- find the procedure code, click add- then ok/post

# Placing Documents in Dentrix Document Center

1.  	From the Dentrix schedule, click on the patients Family File on the upper left of the toolbar. Type in the desired patients name and click on Dentrix Document Center that looks like a filing cabinet on the toolbar of patient’s Family File.

a.   	A window will pop up asking if you would like to set up the Automatic File Acquisition. Click No.

2.  	Once you have the Document Center on the screen, click the paper with a blue arrow to import the patients PDF file to Document Center.

3.  	The file folder will come up to click on the PDF for that patient. Make sure you are in the correct file (Medical History or Ashley), and click the PDF to be transported to the Document Center.

4.  	A window for the Document Center will pop up and choose the document type for the PDF being placed and input the date as the date on the paperwork. The description can be used to go into more detail or retatse what the Document Type says. When finished press ‘ok’ and repeat steps until finished or patient changes.

# PATIENT EDUCATION

From the treatment planner you can select the presentation button- (blue circle)

This allows the patient to watch short videos or photos of their treatment plan if you click on the patient education tab and then pressing play. (like consult pro)

You’ll see that some items are videos and some just photos. This is patient specific and only has videos for the codes in the patient’s treatment plan

You can draw on the photo or video if necessary to show the patient their exact area. You can also take a screenshot if needed.

# Scanning MH and Patient Paperwork

A. There are two colored folders by the printer

1.  Purple folder is for Medical Histories and Medications lists.

· Make sure the patients name is clearly printed or written. If the patient has written in light ink, you may need to go over in dark ink so that the writing is legible when scanned.

2.  Green folder is for any other paperwork pertaining to the patients.

· Example, EOBs, HIPPA, Fax requests, FA’s, Consents, and etc.

.  Please ensure the names are legible for all paperwork

B. 	Begin scanning paperwork in the green folder. Check to make sure that any paperwork belonging to the same patient is paper clipped (staples must be removed), so they can be scanned at the same time.

C. 	To begin scanning, place the document or documents facing up, with the top of the paper(s) to the left on top of the RICOH printer.

1.  Beginning from the home screen on the printer.

2.  Click ‘Scanner’

3.  To scan documents from the Green folder, click ‘Ashley’

· If the document includes more than one page, click ‘divide’ on the left side of the printing screen.

·  For scanning multiple pages into the same file, coming from the previous step, click the ‘divide’ button on the right of the previous screen and press ‘cancel’ when it pops up to remove the setting of dividing each page into a different PDF file. Proceed to scanning documents.

·  To return to the original setting of separating each document, click the ‘divide’ button again and press ‘ok’ when it pops up. Proceed to scanning documents.

·  To scan a two sided document, click the ‘divide’ button on the left side from the main scanning screen and click the button that says ‘two sided original’. Press ‘ok’ button to continue and begin scanning.

·  Some files will have two sided documents and include multiple pages, for that incorporate the previous steps if needed and scan documents.

4.  To scan documents in the Purple folder, click ‘Medical History’

·  Follow the same steps as stated for scanning from the green folder if needed.

# Updating Patient Files in Dentrix from DSN

How to view your patient before they arrive to assure they have all converted data (chart notes in the correct order, SS#, cell#, alerts and special notes)  Have DSN open also:

A.  If pt’s cell phone number is missing:

1.  Click on Family file icon

2.  Double click on top portion (demographics)

3.  look in DSN to see if pt has a cell#, if so copy (control C) cell# and go back to Dentrix demographics and paste (control V) cell # in correct block.

B.  If SS# is missing:

1.  Go to Family file under pt notes and click view button, DSN saved SS# in this section, highlight SS# and cut(control X), click into the demographics section and paste (control V) in correct block.  If there are no other notes, click clear and then ok.

C.  Correct Medical Alerts:

1.  While in Family file, click in upper icon bar on the blue guy with the yellow flag(if flag is white, pt doesn’t have any medical alerts)

2.  Click edit, if the all box is checked, uncheck it, then check the patient chart and prescriptions.

3.   In the box to the left, there will be a list of alerts, if was copied from DSN it is duplicated and spaced incorrectly.  Delete the duplications and then realign each of the alerts.   In the options box, uncheck Display Aged Balance.  Also, copy medical alerts, click on the red cross (or clear cross if no alerts are entered), click edit and paste on the right side under notes.  If any of the medical alerts are included on the list on the left, highlight the alert on the left and then remove the same alert from the right.

D.   Clinical Notes:

1.  Now go to clinical chart – click on clinical notes, if there is nothing from DSN progress notes, go to DSN, open progress notes for pt.   Determine how many notes you will be copying IE:  if past 11 pages, you will need to copy it in sections.

2.  Start at top of notes and copy (control C) the first 11 pages

3. Go to Dentrix clinic chart, click on new clinical note and paste (control V), click on the icon to change date and change the date to the oldest date at the end of the 11 pages.

4. Continue this pattern going from oldest to newest until all notes have been copied to Dentrix.

E.  Also review in DSN the face of the appts to transfer any info such as  "HM", "Hates to see needles" , " Only RSC", No Charge for PM, Collect, etc.

1.  Depending on if note pertains to only 1 appt, add it to the appt by double clicking on appt and adding to the appointment description block.

2.  If note pertains to all appts for pt, copy note and create a new alert under the blue man with the flag.  Title the new alert with description and check the areas that you want it to appear.  Add description in the note section.

F.   Insurance

1.  Review in DSN if pt has insurance, if so add pt to a list that will be given to ESS.

G.  How to change continuing care:

1.  Click on Family file – Make sure there isn’t both perio and prophy

2.  If there is – Go to DSN and determine if the pt is a prophy or PM, go back to the Family file, double click on continuing care,  click on the tx that the pt  doesn’t receive and click the clear button, another box will appear, click yes you are sure you want to clear and then close.

Pictures for Reference